3GPP TSG-SA WG2 Meeting #171	S2-2508687

Wuhan, China, Oct 13-17, 2025

Source:	Ericsson

Title:	KI#1, New Sol: Non-IP/UDP transport for IMS voice media using IP-type PDN Connection

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: The contribution proposes a new solution to enable IMS voice over NB-IoT(GEO) in EPS.

# 1. Introduction

This contribution proposes a new solution to Key Issue #1:

-	Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

This new solution uses similar principle as solution#3, but with differences and details:

-	The UE capability exchange for the support of voice media by non-IP data between UE and network happens in PDN connection establishment, not in attach.

-	The UE capability exchange is handled by PGW-C, but not MME.

-	Providing details on how PGW performs the IP/non-IP conversion, and clarifies the header handled by PGW is only IP/UDP, but not RTP header.

-	Providing details on how UE handles the non-IP transport for voice media, and extending the TFT function for header handling, and also clarifying that the non-IP data sent out of UE is without IP/UDP. The (new) RTP header (the new design is up to SA4) is kept in the non-IP data and is transparent to EPC.

# 2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-19.

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 | X |  |  |  |  |
| #3 | X |  |  |  |  |
| #4 | X | X | X | X |  |
| #5 | X |  |  |  |  |
| #6 | X |  |  |  |  |
| #7 | X | X |  |  |  |
| #8 | X |  |  |  |  |
| #9 | X |  |  |  |  |
| #10 | X |  |  |  |  |
| #11 | X |  |  |  |  |
| #12 |  | X |  |  |  |
| #13 |  | X |  |  |  |
| #14 |  | X |  |  |  |
| #15 |  | X |  |  |  |
| #16 |  | X |  |  |  |
| #17 |  | X |  |  |  |
| #18 |  | X |  |  |  |
| #19 |  | X |  |  |  |
| #20 |  | X |  |  |  |
| #x | X |  |  |  |  |

* * * Next change all new* * * *

## 6.X	Solution #X: Non-IP/UDP transport for IMS voice media using IP-type PDN Connection

### 6.X.0	High-level solution Principles

In case of strictly limited bandwidth (e.g. 1kbps) in GEO satellite link, transport of the voice media traffic without IP/UDP header may reduce the protocol overhead and improve the voice quality, providing that the overhead of RTP header is also reduced.

NOTE:	The terminology ‘non-IP data’ used in this paper only refers to packet without IP/UDP header.

Editor’s note:	It is FFS and up to SA4 whether and how to reduce the overhead in the RTP header.

Editor’s note:	With new RTP header design, the new RTP header may not be subject to ROHC but will be sent as e2e payload between IMS AGW and UE. The IP/UDP headers can be compressed by ROHC profile IP/UDP (0x0002). It is FFS whether the new RTP header design can mitigate the potential protocol overhead issues when applying the existing IP based VoLTE solution to GEO NB-IoT link, for example whether the ROHC header size when using IP/UDP profile (0x0002) plus the new reduced RTP header can lead to less protocol overhead variation in different scenarios (e.g. talk, silent, etc).

This solution addresses Key Issue #1 and covers the aspects how to integrate the new function of non-IP transport with existing VoLTE solution. Other aspects are to be addressed by other solutions.

The solution principle is similar to the principle of Solution #3:

- 	One IP type PDN connection established for IMS, using UP

-	SIP messages are transported using default EPS bearer

-	Voice media is transported by non-IP data between UE and PGW, using a dedicated EPS bearer of the IP type PDN connection

-	Voice media is transported by IP data as for VoLTE between PGW and IMS-AGW

Figure 6.X.0.1 EPS bearers for IMS over NB-IoT (GEO)

### 6.X.1	Description

In this proposal, PDN Connection to IMS APN, with IP type and two bearers, is used for SIP signalling and voice media. The SIP signalling is delivered via default bearer with IP data, and voice media traffic is delived via dedicated bearer using non-IP data between UE and PGW.

### 6.X.2	Procedures

The UE establishes IP type PDN Connection for IMS APN and indicates its support of transporting voice media traffic without IP/UDP header over a dedicated bearer in the PDN connectivity request message, in PCO IE. MME and SGW forwards the UE’s capability to PGW via Create Session Request message.

When an IMS voice call is initiated, the PGW receives the RAR (Policy and Charging Rules Provision) from PCRF as a trigger to establish a dedicated bearer. The PGW initiates the dedicated bearer activation procedure and informs the UE via SGW/MME that the dedicated bearer is for transporting voice media traffic without IP/UDP header. The IMS voice traffic is handled as follows:

-	For UL voice traffic, the PGW adds the IP/UDP header based on the IP flow description that it has received in PCC rule from PCRF.

-	For DL voice traffic, the PGW matches the IP flow using downlink packet filter and associates the traffic with the dedicated bearer but removes the IP/UDP header before transmiting the packet onto the GTP tunnel.

- 	The RTP header is carried transparently via the PGW in both UL and DL.

-	For UL voice traffic, the UE is provided by PGW with uplink packet filter in TFT and an associated indicator. This allows the UE to associate the non-IP data with the dedicated bearer or convert IP to non-IP before transmitting to the dedicated bearer, depending on the UE internal logic.

-	For DL voice traffic, the UE is provided by PGW with downlink packet filter in TFT and an associated indicator. The UE receives DL packet without IP/UDP header from the dedicated EPS bearer, and then the non-IP data may be forwarded to upper layer application directly or converted to IP data first, depending on the UE internal logic.

Figure 6.X.2.1 Voice media transport without IP/UDP header

### 6.X.3	Impacts on Services, Entities and Interfaces

UE:

- Indicates in PDN connectivity request its capability of transporting voice media traffic without IP/UDP header.

- Sending and receiving IMS voice media as non-IP data, per indication from the network.

PGW:

- PGW-C/PGW-U activate dedicated bearer for transporting voice media traffic without IP/UDP header.

- PGW-C provides indication associated with TFT for UE to handle non-IP data over a dedicated bearer.

- PGW-C instructs PGW-U how to add/remove IP/UDP header for dedicated bearer of IP type PDN.

- PGW-U performs IP/UDP header removal/addition when receiving and sending data from/to SGi for dedicated bearer of IP type PDN Connection.

MME/SGW:

-	Support NB-IoT with dedicated bearer.

-	Support dedicated bearer of an IP type PDN connection for transporting non-IP data.

RAN:

-	support NB-IoT with dedicated bearer.

-	support radio bearer which is used for IMS voice media to transport non-IP data.

* * * End of change * * * *