# SA WG2 Meeting #170, 																                  S2-2509249

Wuhan, China, October 13 - 17, 2025

Source:	Lenovo

Title:	Update solution 30 to reflect changes in heart-beat timerS2-2509249

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract of the contribution: This paper proposes an update for solution 30 related to heartbeat timer.

# 1.	 Discussion

The update proposal aims to introduces the appropriate modifications needed in the heartbeat timer when an NF enters an energy saving state and is re-empowered again.

# 2. Proposals

It is proposed to adopt the following text in TR.23.700-67v0.2.0.

* * * Start of 1st Change * * * *

## 6.30	Solution #X: Using the NF Status to control NF traffic before activating energy saving state

### 6.30.1	High-level solution principles

### This solution proposes the following principles to be agreed:

### Avoid discovering an NF in the 5G core, which is selected by the OAM to enter an energy saving state (as per TS 28.310) in the near future. For instance, avoid using a UPF that is selected by the OAM to enter an energy saving state as soon as it can finalize handling ongoing user plane data traffic or offload ongoing user plane data traffic to another UPF.

### Limit the energy usage of an NF, e.g., by restricting the user plane data traffic related to a UPF, or enhance the NF energy efficiency or the use of renewable energy.

- Update the heartbeat timer of the NF to avoid triggering restoration procedures once an NF enter an energy saving state.

## 6.30.2	Description

This solution introduces a mechanism based on existing the NF status parameters contained in the NF profile enabling an NF selected by the OAM to be powered off, i.e., to enter an EnergySaving state, to handle ongoing traffic without disrupting an ongoing service. This can be accomplished by allowing ongoing traffic, e.g., to be transferred to another NF, or allowing an ongoing session to be completed, assuring in this way that an ongoing service is not disrupted when an NF is powered off. This solution can be complementary to solution 20.

In particular, this solution assumes that the OAM configures the energy saving states of the network, i.e., EnergySaving /Not EnergySaving State as per TS 28.310. Once the OAM identifies and selects an NF to enter EnergySaving State, it will soon be powered off, i.e., not be able to handle any more user requests or user pane data traffic.

However, such NFs, e.g., UPFs, may already handle user data traffic, typically small amounts. At the same time the operator needs to make sure that such UPF is not further selected to handle any new request or user data traffic. To assure that there is no service disruption, there is a need to control that an NF that is going to enter an EnergySaving State, handles no user requests or user data traffic at the time instance that is powered off.

Currently, the NF Profile in TS 29.510 contains the NF Status parameter, which is responsible to provide information related to the operational status of the NF. NF Status information contains three operative status information, i.e., in where the NF is on Not EnergySaving State, including "REGISTERED", "UNDISCOVERABLE" and one not operative status information "SUSPENDED" that deals with failures, e.g., as a result of a NF Heart-Beat failure and may trigger restoration procedures (see clause 6.2 of 3GPP TS 23.527).

In addition, the SHUTDOWN TIME it represents a scheduled shut down related only to the NF Status "UNDISCOVERABLE", i.e., it is present if the NF Status is set to "UNDISCOVERABLE". It represents the time until when an NF would be on "UNDISCOVERABLE" status, until it is shut down.

Although, the NF Status "UNDISCOVERABLE" cannot be adopted to represent an NF in EnergySaving State because it is inherently defined to represent an operative NF, it can be used for assuring that an NF selected and scheduled to be on an EnergySaving State is still able to process requests for existing resources or sessions but cannot accept new resource creation or session establishment. The respective SHUTDOWN TIME it can also complement "UNDISCOVERABLE" NF Status by indicating a timestamp when the NF Instance is planned to be shut down forcing the NF to offload current context or user data traffic. When a NF is shut down due to energy related reasons it needs to be reflected by indicating that the NF is on EnergySaving State.

If the SHUTDOWN TIME is not present, then the NF shall either wait for the ongoing user data traffic to be terminated or be offloaded before it can be shut down, i.e. enter an EnergySaving State.

In other words, the SHUTDOWN TIME cannot replace the Schedule of EnergySaving/not EnergySaving proposed in solution 20, because it contains information related to powering off an NF and not powering it on again. Hence, it can serve as a complementary timer relate to indicating the exact time of powering off an NF.

The NF Status can also be used to limit the NF energy cost (i.e., consumption, or carbon emissions) or enhance the NF energy efficiency or the use of renewable energy by allowing an NF to be selected only by limited other NF types or NF types that belong to e.g., a certain slice, or with respect to a certain slice by adopting the "CANARY_RELEASE" and controlling the related selection conditions attribute.

A complementary issue is to fine tune the Heart-Beat timer related to the NF on EnergySaving state. If the original timer is kept, an NF on EnergySaving state will not be able to provide the required keep alive message and the NRF will be misled that the NF failed, triggering restoration procedures. To avoid this issue the NF in EnergySaving state shall configure the Heart-Beat timer to be aligned with the next time it will be on a Not EnergySaving state, i.e., with a REGISTERED NF status, and this need to be according to the energy saving schedule. Configuring the Heart-Beat timer though is NRF specific and not visible to the NF consumer. This means that fine tuning the Heart-Beat cannot replace the need of a schedule for alternating between EnergySaving and Not EnergySaving states.

### 6.30.2	Procedures

The procedures are not impacted but the NF profile information is impacted.

### 6.30.3	Impacts on Services, Entities and Interfaces

The impacts are:

NRF:

-	support the EnergySaving /Not EnergySaving State in the NF Profile.

-	enhancement of NRF logic to enable the indication of EnergySaving State when the "UNDISCOVERABLE" NF Status cause an NF to be shut down because of energy related reasons.

-	update the value of the heartbeat to match the one provided by the energy saving/not energy saving state

OAM:

-	support the enhancements to handle the changes introduced in the "UNDISCOVERABLE" NF Status.

* * * * End of Changes * * * *