3GPP TSG-SA WG2 Meeting #171	S2-2509344

Wuhan, China, 13 – 17 October 2025											 (revision of S2-2508678)

Source:	Ericsson

Title:	Architecture requirements and assumptions updates

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes updates to clause 4 architecture requirements and assumptions for the supporting of SMS delivery to emergency centre

# 1.	Discussion

The agreed SID for the support for SMF to emergency centre in SP-250837 has some statement regarding the baseline of the work, copied below.

# 4 Objective The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

The highlighted text in the SID above indicates that SMS from Emergency Response Centre is delivered as regular short message. This means for UE to receives SMS from Emergency Response Centre, the UE is registered and allowed/authorized to have SMS service in the subscription. This needs to be reflected in the clause 4 of TR 23.700-05

In addition, we have the following EN needs to be addressed in clause 4 of TR 23.700-05:

Editor’s note:	Whether the delivery of SMS to Emergency Response Centre shall be exclusively via an SMSC is FFS.

The highlighted text in the SID above indicates that existing architecture shall be followed. This means the SMS service centre is responsible for the Short Message Transfer Layer and Application Layer (SM-TL/SM-AL) and final delivery to Emergency Response Centre. Othe entities in the delivery chain, e.g. MME/SMSF, supports only the Short Message Relay layer (SM-RL).

# 2.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 4.1	Architectural Assumptions

Editor's note:	This clause documents the common architecture assumptions identified for the study.

-	The architectures for SMS over NAS (see TS 23.272 [7], TS 23.401 [5], TS 23.501 [2], TS 23.502 [3]) and SMS over IP (see TS 23.204 [8]), and the architectural principles of Emergency Services (see TS 23.167 [9]) will be used as a basis.

-	SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

-	The Short Message Service to Emergency Response Centre shall be supported also for roaming UEs, where roaming subscribers’ SMS is routed to a local PSAP, according to the local regulations of the UE’s location.

-	For SMS over NAS over EPS, 3GPP access (including TN and NTN) is in scope.

-	For SMS over NAS over 5GS, 3GPP access (including TN and NTN) and non-3GPP access are in scope.

-	UEs in limited service state are not in scope of this document.

-	Short Messages initiated by the Emergency Response Centre in response to a Short Message to the Emergency Response Centre sent from the UE are treated as regular Short Messages.

-	Short Messages can be routed to different Emergency Response Centres, depending on the type of emergency service.

-	The identification of the type of emergency service (police, ambulance, fire brigade, etc.) is based on the same mechanism used for emergency calls (see TS 23.167 [9]).

-	This study builds upon requirements specified in clause 36 of TS 22.101 [6]. For SMS over IP emergency routing, existing IMS emergency services architecture in TS 23.167 [9] shall be leveraged.

-	The payload structure of the SMS to Emergency Response Centre should be consistent across SMS over NAS and SMS over IP transport mechanisms to ensure uniform PSAP processing capabilities.

-	Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages.

-	The final delivery for MO SMS to Emergency Response Centre over NAS is handled by SMS service centre.

* * * End of change * * * *