SA WG2 Meeting #172	S2-2509968

17 - 21 November, 2025, Dallas, USA

Title:	[DRAFT] Reply LS on Multiple Notification targets for NF Event Exposure services

Response to:	LS on Multiple Notification targets for NF Event Exposure services (S2-2509861 / C3-254606)

Release:	Rel-17

Work Item:	eNA_Ph2

Source:	Nokia [will be SA2]

To:	CT3, CT4

CC:	-

Contact person:	Yannick Lair

yannick.lair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks CT3 for their LS in on Multiple Notification targets for NF Event Exposure services.

CT3 asks the following question:

Question:

When the DCCF receives a subscription with multiple notification target addresses, may the DCCF send the multiple notification target addresses in the subscription request towards the data source?

Answer from SA2:

DCCF is responsible for coordinating data collection and avoiding repeating subscriptions to data source when multiple data consumers are looking for the same data. If the consumer of DCCF services provides multiple notification target addresses, then DCCF will forward the notifications from data source to all relevant notification endpoints. This is specified in 3GPP TS 23.288, see e.g. excerpt below from clause 5A.3.1:

### 5A.3.1	Data Delivery via the DCCF or NWDAF

Figure 5A.3.1-1: Data Delivery via DCCF

Data Delivery via DCCF is shown in Figure 5A.3.1-1. Each Event Notification received from a Data Source NF is sent to the DCCF which propagates it to all Data Consumers / Notification Endpoints specified by the Data Consumers or determined by the DCCF. Each Data Consumer may specify in its request to the DCCF multiple notification endpoints, which may include the requesting Data Consumer, an ADRF or other NFs. The DCCF may also select an ADRF or other notification endpoint based on configuration. The DCCF supports formatting and processing for each Consumer / notification endpoint so notifications comply with the data requests received from each Consumer NF.

Upon the DCCF determining the status of data collection for a data request (see clause 5A.2):

-	If the requested data is not already being collected from a Data Source, the DCCF sends a new subscription/request towards the Data Source with the notification target specified as the DCCF.

-	If the requested data is partially covered by existing subscriptions with the Data Source, the DCCF sends to the existing Data Source a request to modify the subscription and/or creates new subscription(s) to new Data Source for the newly requested data which cannot be provided by the current Data Source.

-	If the requested data is already being collected from the Data Source, the DCCF determines that no subscriptions to the Data Source need to be created or modified.

[…]

Therefore, SA2 answer to CT3 question is that DCCF does not send multiple notification target addresses in the subscription request to the data source when the DCCF receives a subscription with multiple notification target addresses.

# 2	Actions

To CT3, CT4

ACTION: 	SA2 kindly asks CT3 and CT4 to take the SA2 answer into account.

# 3	Dates of next SA WG2 meetings

SA2#173   				09 – 13 February, 2026			              Goa, India

SA2#174   				13 – 17 April, 2026			              Malta