3GPP TSG-SA2 Meeting #172	S2-2510121

Dallas, USA, November 17 – November 21, 2025

Source:	vivo

Title:	[General aspects] Architectural Assumptions and Requirements

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes to update the Architecture Assumptions and Architectural Requirements.

Discussion

Regards to the architecture requirements in S2-2509778, this paper proposes to consider the possible system level aspect to support the RAN defined or RAN led features. For example, the RAN SID proposal (FS_6G_RAN_Scen_Req) includes the following objective:

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: Energy efficiency and energy saving: both for network and device.

For the above bullet in the RAN 6G SID, it requires to ensure the elevated user experience in 6G, while increasing the energy efficiency and energy saving for both networks and devices. Since RAN mainly focuses on the radio enhancements for the energy related features, SA2 may need to study the system level impacts to support the energy related features defined by RAN WGs.

Therefore, this contribution proposes that our SA2 work should align the progress of the features defined or led by RAN groups (e.g., energy related features), if needed. The NOTE should be added as the placeholder for the possible future work accordingly.

Proposal 1: Regards to the Architecture Requirement, it is proposed to add a NOTE in the Architecture Requirement to clarify that the possible future work may be needed to study the system impact to support the RAN defined or RAN led features accordingly.

Regards to the architecture assumptions in S2-2509777, we think that the system architecture design should avoid duplication of functionality (e.g., Tracking area/ RAN Notification Area, mobility management) between 6G RAN and 6G CN. This is an important assumption we should follow for our design and solutions. So we propose to keep the related bullet 3b and add some examples for clarification.

Proposal 2: Regards to the Architecture Assumptions, it is proposed to keep bullet 3b and add some examples in the bullet for clarification.

Proposal

It is proposed to agree the following changes based on the S2-2509778 and S2-2509777 provided by penholder at SA2#171 meeting.

# * * * * Start of 1st Change (All New Texts, changes shown w.r.t S2-2509778 by penholder) * * * *

4.2	Architectural Requirements

The following architectural requirements are considered:

// Option 1

1.	The 6G System supports roaming.

2.	The 6G System considers the following aspects: cloud nativeness, sustainability and energy efficiency, robustness and resiliency.

3.	The 6G System interfaces specified based on study conclusion aim to support multi-vendor interoperable interfaces.

4.	The 6G System aims to minimize inter dependencies between NFs.

12.	The 6G System aims to be predictable for all services offered by 6G System (e.g. when a user wants connectivity, the user gets connectivity).

NOTE: The system level aspects in support of the features defined in RAN WGs (e.g., energy related features) should be considered, if any.

// Option 2

1.	The 6G System shall support roaming.

2.	The 6G System should consider the following aspects: cloud nativeness, sustainability and energy efficiency, robustness and resiliency.

3.	The 6G System interfaces specified based on study conclusion shall support multi-vendor interoperable interfaces.

4.	The 6G System should minimize inter dependencies between NFs.

12.	The 6G System shall be predictable for all services offered by 6G System (e.g. when a user wants connectivity, the user gets connectivity).

NOTE: The system level aspects in support of the features defined in RAN WGs (e.g., energy related features) should be considered, if any.

# * * * * Start of 2nd Change (All New Texts, changes shown w.r.t S2-2509777 by penholder) * * * *

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2. 	This study assumes that control and user plane of the 6G RAN connects to a single core network type, i.e. 6G CN. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

NOTE 1: This does not exclude network sharing scenarios such as Multi-Operator Core Network.

NOTE 2: Additional migration options, if included by TSG RAN as per [ref: RP-252912], beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied (if needed) in alignment with TSG RAN timeline.

3.	The 6G system is assumed to maintain the RAN and CN functionality split as in 5GS.

3b.	The study aims to avoid duplication of functionality (e.g., Tracking area/RAN Notification Area, mobility management) between 6G RAN and 6G CN.

4.	Voice and video services will be provided by IMS.

6. 	The 6G system is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

# * * * * End of Changes * * * *