3GPP TSG-SA2 Meeting #172	S2-2510171

Dallas, TX, USA, 17-21 November 2025

Source:	Nokia

Title:	[KI#3] Conclusions proposal for KI#3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to proceed with some conclusions for normative work for KI#3.

# 1.	Introduction

At SA2#171 some principles were agreed. This paper aims at documenting a set of conclusions for KI#3 as a basis for normative WID drafting at SA2#172.

# 2. Conclusion

It is proposed that the following text is agreed for TR 23.700-67

Start of changes

# 8	Conclusions

As a result of the study documented in this TR, the following are to be specified as part of rel-20 normative specifications:

For KI#3:

For NF selection, the NF profile is enhanced to include the parameters in the table below.

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses theEnergy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

End of changes