3GPP TSG-SA WG2 Meeting #172	S2-2510180

Dallas, USA, November 17 – 21, 2025

Source:	NTT DOCOMO

Title:	[WT#5] Data framework for 6G network

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes scope and key issues of the “WT#5: Study data framework for all aspects related to efficient and scalable data handling including” of the 6G SID.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT and KI.

Data labelling for data management facilitation
Data handled by the data framework is expected to underpin a wide range of AI services and operations in the 6G era. For example, data collected and stored in the CN can serve not only CN functions but also external operator services. To handle the many new services expected in the 2030s, the data framework must be scalable. This implies storing and exposing data in generic and reusable formats so that collected datasets can be easily consumed across use cases. We propose data labelling as a key enabler: by attaching consistent, task-agnostic labels, stored data becomes well-structured and readily usable for diverse applications.

Data type studied in SA2 data collection framework
TSG SA provided guidance on the division of work between SA2 and SA5 in SP-251261. According to this guidance, SA2 and SA5 are requested to consider the 5G work scope as the starting study scope in each WG. In our understanding, SA2 is responsible for data processed/used in the CN (e.g., sensing, positioning, data for AI in CN), while SA5 is responsible for management data

Requirement of data collection framework
In 6G, many use cases require data collection in the CN (e.g., sensing, positioning, AI in CN). Depending on the use case, the architectural requirements for data collection vary. To design the 6G data framework, SA2 should identify the requirements of data collection per use case and assess whether existing mechanisms can meet them. Based on this analysis, necessary enhancements to the 6G data collection framework can be derived.
Different WTs (e.g., WT#3, WT#4) discuss use cases that may require data collection. Hence, it is reasonable for each WT to discuss the data collection requirements that correspond to their considered use cases. On the other hand, WT#5 focuses on the data collection framework.

Common data collection mechanism
In TR 22.870, SA1 has stated that the 6G system shall support a common data framework that can address the needs of multiple use cases. In this regard, SA2 should study whether, and how, a common data collection mechanism across multiple use cases can be introduced in 6G.

MNO visibility
MNO visibility is an important aspect of data collection. When data is collected for use/processing in the CN, the CN needs to be aware of what data is being collected. Furthermore, when data is exposed to third parties from the network, the MNO must know what data is being exposed, from security, user privacy, and proprietary perspectives. Such MNO visibility is required regardless of whether the data is standardized or not.

Data governance, privacy protection
There is an inherent trade-off between maximizing data utilization and ensuring robust data governance and privacy protection. Overly strict governance may hinder the efficient use of collected data for service innovation and network optimization, whereas insufficient governance may lead to serious risks, including privacy breaches, security vulnerabilities, and loss of user trust. Hence, an appropriate balance must be achieved.

Fundamentally, data governance and privacy protection are subject to regulatory requirements. The 6G system shall, at a minimum, comply with such requirements while also enabling convenient and effective data usage from a service perspective. As these requirements differ across regions, the 6G data framework should support governance and privacy protection mechanisms that are sufficiently flexible to accommodate national and regional regulations, while still facilitating efficient data utilization. This principle is aligned with TR 22.870, which specifies: “Subject to national or regional regulatory requirement, the 6G system shall provide user privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and for the corresponding information exposure to an authorized 3rd party”.

# 2. Proposal

It is proposed to approve the following Key issues in TR 23.801.

**** First Change ****

# A.X 	Work Task X: 6G unified data framework

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

NOTE 2: The coordination with other WTs for specific use cases is needed.

The WT includes the following aspects:

- Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing, Positioning) potentially subject to the data framework.

NOTE 3: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions.

NOTE 4: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

- Study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution) )

- data labelling (e.g., data structuring to facilitate the data management)/metadata handling

- data processing (e.g., data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 5:	Coordination with SA5 may be required for charging aspects.

NOTE 6:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

- Study system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system in order to satisfy national or regional regulatory.

NOTE 8:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** End of Changes ****