3GPP TSG-SA WG2#172	S2-2510404

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	KI#2: Evaluation and interim conclusions

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Evaluation and interim conclusions on KI#2: IMS enhancement for GEO NB-IoT NTN access.

# 1.	Discussion

## 1.1	Way forward of conclusions on KI#2

For KI#2, two categories of signalling optimization methods for IMS enhancement in GEO NB-IoT are outlined in the Technical Report:

- Text-based: Use SIP protocol as baseline with minimized message size,  message number and some use compression techniques like SIGCOMP (e.g., solution #12, #13, #14, #15, #16, #17).

- Binary-based: Replace text based SIP encoding with binary formats, such as I1-like protocol or other binary formats, to reduce message size (e.g., solution #4, #18, #19, #20).

The text-based methods use the SIP protocol, which requires minimal changes for UE and IMS network support. However, even with optimization, message sizes remain relatively large (e.g., 500–1000 bytes for INVITE).

The binary-based methods are capable of minimizing message size to approximately 50-100 bytes. Nevertheless, implementation requires significant changes to both the User Equipment (UE) and the network to accommodate a new protocol suite alongside SIP.

Call Setup Time (CST) is defined as the time from the end-user initiating a call to the moment the called party starts ringing. The SA1 requirements for CST in case the call is performed over NB-IoT NTN GEO is 30 secs nevertheless other sources like ITU-T E.807 [1] indicate that CSTs (e.g., >6 seconds) may lead to user dissatisfaction, especially in modern networks where expectations are shaped by fast app-based communication. In typical VoLTE that is widely deployed the typical range for CST is 1-4 secs. It is therefore accurate to say that the SA1 requirement of 30sec is wide off the mark of existing user experience for CST over existing mobile radio technologies e.g. VoLTE, VoNR, 2/3G with CSFB.

Observation 1: Nevertheless, each method presents distinct advantages and disadvantages, making it challenging to recommend a single approach in the conclusion of the TR. It is therefore recommended to standardise both options and leave the selection of method for addressing KI#2 up to implementation.

Proposal 1: The text-based method is recommended as one approach for IMS signalling optimization, provided that offers compliance with the existing SIP protocol as specified by IETF and can be used if CST as close as possible to what is experienced by existing mobile radio technologies is not of primary importance.

Proposal 2: The binary-based method is recommended may be considered as another option to achieve improved performance. UE that supports the binary-based method should indicate and negotiate this capability with the IMS network prior to session establishment.

Proposal 3: When accessing GEO NB-IoT, the UE registers with the serving IMS network using text-based SIP. The UE indicates support for text-based and/or binary-based methods and negotiates the chosen signaling method with the network during the IMS Register procedure.

## 1.2	Roaming scenarios for voice over IMS with local breakout

Although the roaming architecture for voice over IMS with local breakout is specified in clause 4.2.3 of TS 23.228, in real world VoLTE and VoNR deployments only utilize S8HR/N9HR based IMS roaming architecture.

In the most typical case for the deployment of IMS voice over NB-IoT NTN, we expect the VPLMN will be a Satellite Network Operator (SNO) and the HPLMN a terrestrial operator (TNO).

If the GEO NB-IoT NTN access is provided by the PLMN which is different from the UE’s HPLMN, roaming architecture of some kind is required to provide voice service over GEO NB-IoT NTN access to the UE. We assume that in typical case GEO NB-IoT NTN access, will be used by the "smartphone" user only if the TN HPLMN or TN VPLMN does not provide access in the area where he/she is in. This use case is expected to be relatively rare. It is therefore considered important that the changes required are as much as possible kept in the VPLMN i.e. SNO network instead of the HPLMN i.e. the TNO network. We therefore suggest that the deployment of interworking capabilities within VPLMN can minimize the impact to IMS network in the HPLMN.

An interworking function may be deployed in the VPLMN (SNO) with GEO NB-IoT NTN access to enable communication with the IMS network in the HPLMN using the SIP protocol. This function operates as P-CSCF*/AGW towards the UE (using either binary or text-based protocol) and acts as a user agent representing the UE when communicating with the IMS in the HPLMN.

Based on the roaming agreement, the possible transcoding between GEO NB-IoT NTN specific voice codec (i.e. ULBC) to the existing voice codec (e.g., AMR) may be performed in the VPLMN or HPLMN.

The example of roaming architecture for GEO NB-IoT NTN access is illustrated in Figure 1.2-1. The Interworking Function is implemented within the VPLMN (SNO) to facilitate the conversion of binary-based signaling to SIP. On one end, it interfaces with the UE as an enhanced P-CSCF, providing support for binary-based signaling. On the opposite end, the Interworking Function serves as a user agent representing the UE, like an IP-PBX as defined in Annex S of TS 23.228 [6], to establish connectivity with the IMS network in the HPLMN. Additionally, the Interworking Function is responsible for proactively translating voice codecs between ULBC and AMR/EVS formats.

Figure 1.2-1: IMS local breakout in VPLMN with GEO NB-IoT NTN access

Figure 1.2-2 illustrates the sample procedures for IMS session establishment under the previously described roaming architecture.

Figure 1.2-2: Procedure of IMS session establishment in roaming  case

0.	The UE sends SIP Register request to Interworking Function in the VPLMN and indicates supporting of binary-based signaling. The IWKF carries out the SIP registration process with the IMS in the HPLMN utilizing the UE's SIP ID. During this procedure, the UE IP address is substituted with the IWKF address.

1.	UE sends a binary INVITE to IWKF.

2.	IWKF translates it to SIP INVITE and forwards it to IMS core in HPLMN.

3.	Codec negotiation and resource reservation occur between IWKF and terminating NW/UE.

4.	Terminating UE sends 180 Ringing to IWKF.

5.	IWKF converts and relays Ring indication to UE in binary.

6.	Terminating UE sends 200 OK to IWKF.

7.	IWKF notifies UE by binary message that the call is answered.

8.	The session is active and ready for voice transmission.

Observation 2: Possible new business models for NTN access, such as GEO NB-IoT access offered by a dedicated MNO, should be taken into account when designing the roaming architectures. The impact to the IMS network in the MNO with TN access should be minimized in this scenario.

Proposal 4: When GEO NB-IoT NTN access is provided by a PLMN different than the UE’s HPLMN and the binary-based method is used between the UE and network, an enhancement to the roaming architecture is required. To minimize the impact on the IMS network in the HPLMN, the VPLMN needs to deploy appropriate interworking capabilities to support the binary-method and interwork towards the HPLMN using SIP. When interworking with IMS in the HPLMN, the interwork function uses the procedure of IP-PBX as defined in Annex S of TS 23.228 [6].

# 3.	References

[1] ITU-T, E.807: Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

## 8.x	Interim conclusions for Key Issue #2

The following interim conclusions for Key Issue #2 IMS enhancement for GEO NB-IoT NTN access are made:

-	The text-based method is selected for normative work for IMS signalling optimization, provided that strict compliance with the SIP protocol as specified by the IETF is maintained.

-	The binary-based method is selected for normative work to achieve improved performance. UE and network that support the binary-based method should indicate and negotiate this capability with the IMS network prior to session establishment.

-	The protocol between UE and network for binary-based method will be decided by CT1.

-	When accessing GEO NB-IoT, the UE registers with the serving IMS network using text-based SIP. The UE indicates support for text-based and/or binary-based methods and negotiates the chosen signaling method with the network during the IMS Register procedure.

-	When GEO NB-IoT NTN access is provided by a PLMN different than the UE’s HPLMN and the binary-based method is used between UE and network, an enhancement to the IMS roaming architecture is required to enable voice services over this access. To minimize the impact on the IMS network in the HPLMN, the VPLMN needs to deploy appropriate interworking capabilities to support the binary-method and interwork towards the HPLMN using SIP.

-	When interworking with IMS in the HPLMN, the interworking function uses procedures of IP-PBX as defined in Annex S of TS 23.228 [6].

# >>>>END OF CHANGES<<<<