3GPP TSG-SA WG2 Meeting #172	S2-2510591

17-21 November, 2025, Dallas , US 	(Revision of S2-250xxxx)

Source:	China Mobile

Title:	Interim conclusion of KI#3

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim conclusion for KI#3 of TR 23.700-56.

1. Discussion

This contribution proposes interim conclusion of KI#3: Supporting dynamic configuration of MDC2 endpoints.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *(All new texts)

7.1.X	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	The DCSF services are introduced to support dynamic configuration of MDC2 endpoints via interactions between DCSF and DCAS;

-	The DC AS implicitly subscribes events of MDC2 connection management, including connection establishment, connection modification and connection release;

-	The DCSF notifies the DC AS the events and MDC2 media endpoint information of the MF and gets MDC2 media endpoint information of the DC AS via the response of the notification from the DC AS.

# * * * * End of changes * * *