SA WG2 Meeting #S2-172	S2-2510896

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510869)

Source: 	vivo

Title: 	KI#1 interim conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update interim agreement for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following changes to TR 23.700-04.

Start of Change

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

1. UP path termination entity

-	An 5GC NF is defined as the UP path termination entity to support the transfer of standardized data over UP for UE data. The UP path termination entity will further transfer the received data to the UMTES

Editor’s NOTE: 	Which 5GC NF will be defined as the UP path termination entity is FFS.

NOTE 1: The UMTES is the AF/OTT server for UE-side model training.

2. Discovery of UP path termination entity

-	(the UMTES discovers UP termination entity) The UP path termination entity may be either locally configured at the UMTES, or the UP path termination entity may be discovered by the UMTES via NRF (if UMTES is trusted) or by NEF (the UMTES is untrusted).

The following options allow the UE to learn the UP path termination entity, either a or b,

- the UE has the UP path termination entity address/FQDN locally configured

- the 5G core network sends UP Path termination entity address (i.e. IP address/FQDN) to the UE in a N1 container via NAS.

NOTE : In order to make sure that the UE and the UMTES connect to the same UP path termination entity, the UE provides the UP path termination entity address to the UMTES using a mechanism outside 3GPP TS, e.g. over the top, or the UE and AF configure the same UP path termination entity address.

3. UP path establishment

-	The UE establishes a new PDU Session, or uses an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, by reusing existing mechanism (e.g. URSP).

4. Data transfer procedure

- 	UMTES sends the data transfer request to UP path termination entity including the information related to target UE(s) for data transfer (e.g. candidate UE list, TAC(s), geographical area), and the identification of the requested data.

Editor’s Note: what information will be used to select target UE(s) for data transfer is FFS.

-	The UP path termination entity performs the user consent check for data collection for model training with UDM by reusing the existing mechanism.

-	The UP path termination entity sends the Data Transfer Request with identification of requested data for the final list of UEs to the UE

Editor’s note: how to send the data transfer request to the UE, i.e. via NAS signalling or UP is FFS.

-	 The UP path termination entity optionally sends information (e.g. requested data) to gNB via AMF, which may assist the gNB to generate data collection configuration to UE.

NOTE 3: whether and what information to assist data collection configuration will be defined by RAN.

Editor’s Note: how to select the UE by CN and whether and how to select the UE by gNB is FFS.

-	The UE reports the collected data to the UP path termination entity via the established UP path, based on the requirements received in the Data Transfer Request from the UP path termination entity.

NOTE 4: The requested data for UE sided model training will be defined in RAN WGs.

NOTE 5: The mechanism of how UE collects data depend on RAN’s decision.

5.	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	Or, the existing packet filter set (e.g., IP address / IPv6 prefix or port number of the UP path termination entity) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

6.	Controllability

-	The UP path termination entity is allowed to start /stop / modify data transfer procedure based on data transfer trigger info from UMTES.

-	The UE may accept / reject transfer request from the UP path termination entity. The UE could also cancel data transferring based on internal determination.

-	The UP path termination entity may send to gNB via AMF an indication that data transfer is allowed for a UE.

7.	Visibility

-	The UP path termination entity could check the reported UE data by comparing them with the specific requirements (e.g., requested data).

-	The UP path termination entity exposes the received UE data that meet the requirements (if needed) to the data consumer (with the authorization of NEF).

NOTE 6: What requirements will be used by UP path termination entity for data check will be based on requirements of RAN.

Next Change

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether and how to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

End of Changes