SA WG2 Meeting #172	S2-2510953

13 – 17 November, 2025, Dallas, USA

Source:	Ericsson, Huawei, HiSilicon

Title:	[KI #2, New Sol] Enhancements to capability exposure framework and IMS AS service to support audio, video capability in addition with DC capability

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3. The solution addresses the expose of IMS capability for eMMTel services (voice/video).

# 1. Discussion

This pCR proposes a solution to the Key Issue #2:”….how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities” by specifying the addition of MMTel media (e.g. audio, video) to the relevant existing service operations to align with the current specification.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * all new change * * * *

# 6	Solutions

## 6.X	Solution #X: Enhancements to capability exposure framework and IMS AS service to support audio, video capability in addition with DC capability

## 6.X.1	High level Solution principles

This solution addresses KI#2.

Release 19 NG_RTC_Ph2 has specified IMS data channel capability exposure procedures and the related services to enables the AF/AS to request the IMS AS via the NEF to create or release an IMS session with DC media or modify the DC media in a specific IMS session. But in some cases, the NF/AF needs to create an IMS session with audio/video media and DC media at the same time.

OMA has specified a RESTful Network API for Audio Call, but it cannot support DC media. If a 3rd party AF/AS wants to create an IMS session with audio and DC media, it has to invoke the OMA API to create a IMS session with audio firstly and then it invokes Nnef_ImsSessionManagement API specified by 3GPP to add DC media in the established IMS session. It means that the 3rd party AF/AS need to utilize two sets of APIs with different styles (OMA style and 3GPP style) and it cannot use both audio and DC media at alerting phase.

To ease the development of the 3rd party AF/AS and enable that audio and DC media can be used at alerting phase, both Nnef_ImsSessionManagement, Nimsas_ImsSessionManagement and Nmf_MediaResourceManagement service specified in Release 19 need to be enhanced to support audio and video media.

The main principles of the solution are to align with and expand on existing specified functionality:

- to add exposure capabilities to introduce support for establishment, modification and release of MMTel media (e.g., audio & video).

- addition of MMTel media (e.g., audio & video) exposure capabilities is in the context of data channel (e.g. enhancing audio/video services with DC capabilities or enhancing DC services with audio/video services capabilities).

NOTE 1 The term MMTEL media refers to media that is not sent through/ via a data channel.

### 6.X.2	Description

The solution specifies enhancements to the IMS exposure framework ensuring that exposure capabilities for session establishment, modification and release are media agnostic, i.e., other MMTel media (e.g., audio, video) are also supported in addition to the existing specified DC media.

The solution caters for the addition of MMTel media (e.g., audio, video) to the existing specified relevant service operations. The proposal builds on aligning with already specified functionality where the establishment, modification and release of MMTel media (e.g., audio, video) is supported in the following services:

- Nimsas_MediaControl_MediaInstruction service operation (see TS 23.228 AA.2.4.3.2)

- Nmf_MRM_Create service operation (see TS 23.228 AA.2.5.2.2).

3GPP stage 3 has also defined support for audio / video (see TS 29.175, for example chapter 6.1.6.3.4).

The extended functionality will for example enable a DC AS to add, modify or release MMTel media (e.g. audio/video) to / from a standalone ADC or enable a person / user to add, modify or release MMTel media (e.g. audio/video) towards DC AS (for DC enabled services).

The IMS AS services that are amended to indicate support of MMTel media (e.g. audio / video) in addition to the existing DC media are described in section 6.X.4 Service definition enhancements.

Support for MMTel media (e.g. audio/video) shall also be included in the yet undefined services on DC3 & DC4 reference point.

The following two example procedures show the addition of MMTel media (e.g. audio and video) in addition to an A2P application data channel, between the UE-1 and DC AS over MDC2. As result, the MMTel media is anchored in the MF

### 6.X.3	Procedures

#### 6.X.3.1  	DC AS initiated IMS audio session addition with a standalone A2P application data channel

Figure 6.X.3.1-1 depicts a signalling flow diagram initiated by DC AS for adding MMTel media (e.g., audio) to a standalone Application Data Channel A2P application data channel with UE A. This signalling flow extends the flow described in TS 23.228 AG.2.2.2

Figure 6.X.3.1-1: Establishment procedure of an adding an IMS audio session with a standalone A2P application data channel

0.  The DC AS decides to add an IMS audio session with an already established standalone A2P application data channel with UE A.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add an IMS audio session as addition to an already created standalone A2P application DC with UE A.

The DC AS sends the session ID (of previously created ADC) and UE A’s IMPU in addition to the media operation set including the parameters for the media type set as “Audio” to NEF. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session creation.

2.	If the NEF authorizes the request, the NEF queries the HSS with Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE A.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-1 requiring to add an IMS audio session. The request includes the session ID (of previously created ADC) and UE A’s IMPU in addition to the media operation set. The media operation set includes the parameters for the media type set as “Audio”, binding info & the MDC2 media endpoint. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session create progress.

4.	The IMS AS-1 validates the user subscription data and checks the IMS audio & DC capability of UE A to determine whether the request should be notified to DCSF-1. If UE A does not have subscription of IMS data channel and MMTel media (e.g., audio), then the request shall be rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-1 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response generated by the IMS AS-1 to the DC AS.

6.	The IMS AS-1 sends a Nimsas_SessionEventControl_Notify request to the DCSF-1 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from the media operation set (including MediaInfoList: MediaType - “Audio”) received in step 3 and other stored parameters in the IMS AS-1 if applicable.

7.	After receiving the session event control notification, the DCSF-1 determines the policy how to process the A2P media audio establishment request based on the related parameters in the notification and/or DCSF-1 service specific policy.

8.	The DCSF-1 determines that the A2P media audio requires the DCAS audio to be anchored in MF-1.

9.	The DCSF-1 instructs the IMS AS-1 to set up the A2P audio by sending a Nimsas_MediaControl_MediaInstruction request.

10.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to allocate audio media resource towards UE A and MDC2 media resource towards the DC AS.

11.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

12. DCSF send a connection (audio) establishment request offer to DCAS including the MDC2 media resource.

13. DCAS accepts the request and provides MDC2 media resource (audio).

14. The DCSF-1 instructs the IMS AS-1 to set up the audio by sending a Nimsas_MediaControl_MediaInstruction request.

15.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to request update of DCAS side media resource information.

16. The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

17. The DCSF-1 returns the Nimsas_SessionEventControl_Notify response to the IMS AS-1.

18.	The IMS AS-1 generates a re-INVITE request in which the SDP offer contains the media information of the audio A2P application required by the DC AS, binding information and the session ID.

The IMS AS-1 sends the re-INVITE request to UE A.

19.	DC media negotiation may be completed through 18X/PRACK/200 OK(PRACK)/200 OK(INVITE)/ACK procedure.

20.	The IMS AS-1 requests MF-1 to update UE-A side media (Audio) resource information.

21.	UE A established audio with DC AS.

22. If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in step 3, the IMS AS-1 notifies the NEF for the progress of the audio session creation with the Nimsas_ImsSessionManagement_Notify request.

23If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 1, the NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify request.

24. The DC AS returns a Nnef_ImsSessionManagement_Notify response to the NEF.

25. The NEF returns a Nimsas_ImsSessionMangement_Notify response to the IMS AS-1.

#### 6.X.3.2	Person-to-Application (P2A) MMTel media (audio) and Application Data Channel Setup

Figure 6.X.3.1-2 depicts a signalling flow diagram for adding an IMS audio session in a in a person to application use case. This signalling flow extends the flow described in TS 23.228 AC.7.2.2-1 where a P2A ADC is setup.

Figure 6.X.3.2-1: Person-to-Application (P2A) audio & application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.a - It is assumed that DC AS has provisioned triggers in IMS AS to be informed of media setup (e.g. audio (setup) media request).

0.b-3.	As in the base signalling procedure (AC.7.2.2-1), steps 0-3 of clause AC.7.2.1 applies. It is assumed that audio A-B has also been established.

NOTE 1:	The SIP re-INVITE (step 1) is initiated by the served users UE (in this case UE#1).

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel / audio establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added Application Data Channel / audio media of the offer takes DC Application Server as target endpoint and requires to anchor in the MF.

6.	The DCSF invokes Nimsas_MediaControl service operation to instruct IMS AS to terminate the media flow of the originating UE to MF. The instruction also includes information to be consumed by the MF that the audio media shall be relayed via the MDC2 interface.

7.	The IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF on application data channel / audio establishment and data channel / audio media resource reservation based on the DC / audio media information received from DCSF.

8.	The IMS AS notifies the MediaControl instruction control response to DCSF.

9.	The DCSF stores the media resource information and sends a P2A (audio / DC) application data channel establishment request (including the MDC2 media resource received from MF) to the DC Application Server via DC3/DC4.

10.	DC Application Server accepts the P2A application data channel establishment request, returning an MDC2 reserved media resource as answer and is prepared for UE#1 traffic through MDC2.

NOTE 2:	Details on how DCSF communicates with the DC Application Server is for study as part of KI#3.

11.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the MF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	DCSF replies to the notification received in step 3.

15-16.	IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include application data channel request in the SDP for the application data channel.

17-19:	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

20.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

21.	DCSF replies to the notification.

22-23.	The IMS AS includes SDP answer for application data channels / audio to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

24.	The originating network P-CSCF executes QoS procedure for application data channel / audio media based on the SDP answer information from the 200 OK response.

25.	CSCF returns the 200 OK response to UE#1.

26.	UE#1 send ACK to the terminating network.

27.	The application data channel / audio between UE#1 and DC Application Server is established via MF. MF forwards data channel / audio traffic between UE#1 and DC Application Server based on MDC2 media point information received in step 9 and 12.

### 6.X.4	Service definition

#### 6.X.4.1	Nnef_ImsSessionManagement service

##### 6.X.4.1.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nnef_ImsSessionManagement API in the following clauses.

##### 6.X.4.1.2	Nnef_ImsSessionManagement_Create service operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.X.2.1.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address of the application layer is indicated.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted

-	if the Operation Type indicates adding or updating the media, the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

#### 6.X.4.2	Nimsas_ImsSessionManagement Service

##### 6.X.4.2.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nimsas_ImsSessionManagement API in the following clauses.

##### 6.X.4.2.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

NOTE 1:	Only data channel media is supported in this Release.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE 2:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional:  Session ID allocated by IMS AS.

##### 6.X.4.2.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### 6.X.4.3	Nmf_MediaResourceManagement (MRM) service

##### 6.X.4.3.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nmf_MediaResourceManagement API in the following clauses.

##### 6.X.4.3.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The Media ID value is set by the consumer.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

NOTE 1:	Different Termination IDs associated with the same Media ID or associated media ID corresponding to the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type, Audio, Video or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "Audio" or "Video", then the Media Function Description incudes:

-	Codecs and Formats;

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Data Channel SCTP Port: This attribute identifies the SCTP port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	Avatar Media Specification are optional depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	associated Media ID: identical to the associated Media ID received from the consumer.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

### 6.X.5	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

NEF:

- Enhance Nnef_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session

DC AS:

- sending establishment, modification and release requests for MMTel Audio / Video.

- sending DC AS-side information for MDC2 configuration for Audio / Video

- receiving MF-side (audio/video) information for MDC2 configuration.

DCSF:

- receiving subscription of the event ExternalSessionCreateEvent (with MediaType=Audio/Video)

- requesting IMS AS for MDC2 (audio/Video) configuration.

- sending MF-side (audio/video) information for MDC2 configuration to DC AS,

- sending request to create DC AS-side for the MDC2 (Audio/Video).

- receiving DC AS-side information for MDC2 (Audio/Video) configuration

requesting IMS-AS for MF (Audio/Video) configuration in e.g., controlling which media transformation the MF has to carry out

IMS AS:

- Enhance Nimsas_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session.

-	receiving MDC2 (Audio/Video) configuration request from DCSF,

-	instructing MF for MDC2 (Audio/Video) configuration.

-	have to re-anchor voice media towards an MF/from MF based on DCSF instructions.

MF:

- Enhance Nmf_MediaResourceManagement API to support creating, updating and removing audio and video media.

- support Codec, formats and relevant audio/video media capabilities

-	receiving MDC2 (Audio/Video) configuration instructions from IMS AS,

-	performing MDC2 (Audio/Video) configuration for DC AS service like e.g. translation & transcription services.

# * * * * End of changes * * *