3GPP TSG-SA WG2 Meeting #172	S2-2510990

Dallas, USA, November 17 – 21, 2025										(revision of S2-2509949)

Source:	CATT, NTT DOCOMO, Lenovo, InterDigital, NEC, Apple, Vivo, Nokia, China Mobile, China Telecom, ZTE, Ericsson, MediaTek Inc., Samsung, Huawei, HiSilicon, Orange

Title:	KI#2 conclusions

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes conclusions for KI#2.

1. Introduction

Based on the interim agreements for KI#2 “Support of service adjustments for the UE” in clause 7.1.2 of TR 23.700-67, conclusions for KI#2 is proposed.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change (all new text) * * * *

8.2	Conclusions for KI#2

The conclusions for KI#2 are as follows:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information will be addressed in the KI#1.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	AM policy control.

-	SM policy control.

NOTE 3:	Based on operator policy, it is required to have the balance between energy saving and user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	BDT policy (re-)negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information.

NOTE 4:	There is no extension to the existing policy parameters.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

NOTE 5:	Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF will be considered during normative phase taking into account the feedback of SA WG5.

NOTE 6:	Existing NEF services can be updated but no new NEF services will be introduced for the AF request.

* * * End of Change * * * *