3GPP TSG-WG SA2 Meeting #172	S2-2511189

Dallas, USA, November 17 – 21, 2025

# Title:	Reply LS on clarification of UAV regulation from SA2

# Response to:	S2-2509928/R3-255899

# Release:	Rel-19

# Work Item:	UAS_Ph3

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	Aleksejs Udalcovs

E-mail Address:	aleksejs.udalcovs@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: CR 0208 TS 23.256

1.	Overall Description

SA2 thanks RAN3 for the Reply LS on clarification of UAV regulation. Regarding the requested clarification on altitude reporting for aerial UEs, i.e.:

“RAN3 seeks for further clarification from SA2 on whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE. If so, RAN3 asks whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE, and whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration.”,

SA2 has discussed the matter on UAV UE’s altitude reporting and reached the agreement in the attached CR. Based on the SA2 specified procedures and considering that a single aerial UE reporting configuration consists of one pair of derived altitude thresholds (i.e., minimum and maximum altitude thresholds) as indicated in the LS(R3-255899), SA2 provides the following clarifications.

RAN3 asks:	“whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE”.

SA2 replies: Aerial UE location reporting configuration shall enable altitude reporting whenever the UE’s altitude meets any of the configured altitude threshold values. It cannot be ruled out that different pairs of altitude thresholds (i.e., minimum and maximum altitude thresholds) are derived by the UAS NF / NEF for each of the TA/Cell/NG-RAN in the assigned trajectory. The assigned trajectory event is described in clause 5.2.2.3.1 of TS 23.502, and the Stage 3 implementation (see clauses 6.2.6.2.35-37 in TS 29.518) allows to set minimum and maximum altitude thresholds specific to a TA/Cell/NG-RAN.

RAN3 asks:	“whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE”.

SA2 replies:	 Possible, under the assumption that the UAS NF / NEF derived different pairs of altitude thresholds for each of the TA/Cell/NG-RAN in the assigned trajectory and the gNB configures the aerial UE at once for the entire flight.

RAN3 asks:	“whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration”

SA2 replies: gNB is expected to report to the AMF whenever the concerned aerial UE reports its altitude information to the gNB in accordance with the relevant configuration (i.e., a pair of altitude thresholds that can be specific to a particular TA/cell/NG-RAN node).

2.	Actions

To RAN3:

ACTION: SA2 respectfully asks RAN3 to take the above information into account and align stage 3 accordingly.

3.	Date of Next TSG SA WG2 Meetings

SA2#173	February 9 – 13, 2026	Goa, India

SA2#174	April 13 – 17, 2026	Malta