3GPP TSG-SA WG2 Meeting #172	S2-2511301

# October 17-21, Dallas, US	rev S2-2510654

Source:	Xiaomi, Apple, vivo, Futurewei, ZTE, DT, Samsung, LGE, CATT, Lenovo, NTT DOCOMO, Ericsson,

Title:	Conclusion for KI#1 System Architecture to Support Sensing

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes the conclusion for KI#1 System Architecture to Support Sensing.

1. Discussion

TS 23.700-14 v1.1.0 has documented the interim agreements for KI#1 System Architecture to Support Sensing as following:

7.1.1 Agreed Principles for KI#1: System Architecture to Support Sensing To support KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI's interim conclusions will be aligned with KI#1 conclusions.

Proposal 1: copy all the interim agreements for KI#1 in clause 7.1.1 into clause 8.1 conclusion for KI#1.

Observation 2: In last SA2#171 meeting, there was no interim agreement achieved regarding whether the AMF is involved for deliver the sensing data and/or sensing control signalling, with following status:

- For sensing data delivery, Majority companies prefer Principle 2Y “AMF is not involved to support Sensing data delivery”, only few companies prefer “AMF is involved to support Sensing data deliver”.

- For Sensing control signalling delivery, 1) some companies prefer “AMF involved to support sensing control signaling; 2) more companies prefer directly connection between SE and SF (i.e. AMF is not involved); 3) some companies strongly against the option to support both.

Principle 2X: Sensing control signalling is exchanged between SE and SF/SCF directly or via AMF, both.

Principle 2Y: AMF is NOT involved to support for Sensing data delivery.

For 5G-A Sensing, it is better to avoid to introduce additional and unnecessary impacts. Sensing data and sensing control signalling are exchanged directly between SE and SF, it can avoid to introduce additional impacts on AMF. And at the same time, it is also compatible for 6G sensing.

Proposal 2: Sensing data is delivered directly between SE and SF (i.e. without AMF involvement). Sensing control signalling is exchanged between SE and SF directly (i.e. without AMF involvement);

Observation 3: regarding to Whether to introduce a new Gateway Sensing Function (GSF)?), there are four potential functionalities that may be performed by new Gateway Sensing function in the TR solutions, as following:

- Authorization for Sensing service consumer.

- Authorization for Sensing service request

- Discovery and selection for SF

- Sensing result exposure to Sensing service consumer

For 5G-A Sensing, it is better to avoid to introduce unnecessary impacts. For 1) it was agreed in KI#2 interim agreement, that NEF performs the authorization service consumer. So, no need to introduce duplicated Gateway Sensing Function to perform the authorization service consumer.

In KI#1 interim agreement, it was agreed that Sensing Function is defined to support sensing service, there is no issue to perform the Authorization for Sensing service request. if GSF is introduced, the additional interface between GSF and SF should be also defined. So don’t see the need or advantage to define new Gateway Sensing Function to perform Authorization for Sensing service request

For discovery and selection for SF, it was also agreed in the interim agreement for KI#3, SF selection based on NEF’s local configuration or NRF query. It also indicates that NEF would perform the discovery the selection of SF. So there is also no need to introduce a new GSF to perform the same functionality as NEF.

For the sensing result exposure, NEF is defined to expose information to AF. There is no issue for NEF to exposure the sensing result for sensing service. So also don’t see the need to introduce a new GSF to expose the sensing result

Considering above all, the four functionalities can be performed by SF/NEF, don’t see the need to introduce one new Gateway Sensing Function.

Proposal 3: No Gateway Sensing Function is needed in this study.

- Principle 5: if AF is outside the trusted domain, NEF is enhanced to support Authorization for Sensing service consumer (as captured in clause 8.2), Discovery and selection for SF (as captured in clause 8.3), and Sensing result exposure to Sensing service consumer (as captured in clause 8.5).

- Principle 6: the functionality of Authorization for Sensing service request is performed by SF (as captured in clause 8.2)

Observation 4, during the discussion in SA3, it is controversial whether the sensing result may contain additional and un-expectation information, e.g., human information in the sensing area besides the requested aerial object.

Proposal 4: to make further clarification as following principle 7.

- Principle 7: Sensing architecture needs to support aerial object (e.g. drone) detection and tracking as defined in TS 22.137 [2]. And the exposed sensing result is restricted for aerial object (e.g. drone) detection and tracking in this study.

2 merged inputs from other contributions

| S2-2509883 | 23.700-14: Proposed Interim Agrements and Conclusions for KI#1 | OPPO |
| --- | --- | --- |
| S2-2509886 | Proposed way forward on contentious issues | OPPO |
| S2-2509908 | 23.700-14: KI#1: Conclusions and Interim Agreements | LG Electronics |
| S2-2509973 | 23.700-14: Proposed agreement of principles for Key Issues 1 | Nokia |
| S2-2509989 | 23.700-14: Proposed Conclusions for KI#1 | CEWiT |
| S2-2510004 | 23.700-14: KI#1 Conclusion | Apple |
| S2-2510035 | 23.700-14: FS_Sensing_ARC -- update on Interim agreements for KI#1 | China Mobile Com. Corporation |
| S2-2510068 | 23.700-14: Proposal for KI#1 conclusion | InterDigital Communications |
| S2-2510088 | 23.700-14: Adding direct P2P interface between NG-RAN and SF as principles for system architecture (KI#1) | Ericsson |
| S2-2510089 | 23.700-14: KI#1, Adding Principles: NEF is enhanced to support sensing result exposure | Ericsson |
| S2-2510133 | 23.700-14: Update the agreed principles and add conclusion for KI#1 | CATT |
| S2-2510195 | 23.700-14: Conclusion proposal to KI#1 System Architecture to Support Sensing | ZTE |
| S2-2510289 | 23.700-14: KI#1, Update interim agreement | Intel |
| S2-2510387 | 23.700-14: Conclusions on KI#1: System Architecture to Support Sensing | vivo |
| S2-2510409 | 23.700-14: KI#1: conclusion update proposal | Huawei, CAICT |
| S2-2510676 | 23.700-14: Interim agreement update and final conclusion for KI#1 | Samsung |
| S2-2510695 | 23.700-14: KI#1: Update to the Interim Agreements | Lenovo |
| S2-2510760 | 23.700-14: FS_Sensing_ARC: Additional principles for KI#1 on system architecture to support sensing | Qualcomm Incorporated |

Observation 1a: two companies propose to use terms “logical functionalities” before SCF and SPF [LGE, CEWiT]. However, in last meeting, the term “logical” was discussed and not agreed, so propose to keep original text.

Observation 1b: two companies propose to define the functionalities for SCF and SPF.

Propose 1b: propose description for SCF and SPF as following:

- The functionality of SCF may support for, e.g., receiving the sensing service request, authorization of the Sensing service request, sending configuration parameters to SE, etc.

- The functionalities of SPF may support for e.g., sensing data collection, sensing result generation, etc.

Observation 1c: one company proposes to introduce the interaction between SFs, which was not discussed officially in last meeting. So, propose to discuss it in this meeting.

Proposal 1c: There may be multiple SFs deployed (e.g., in various geographical areas) and one SF can communicate and use one or more services of another SF.

(Whether involve AMF)

Observation 2a: For control signalling delivered between SE and SF, the following proposals from companies as:

- 13 companies supporting for “Direct connection without AMF involvement” [OPPO, LGE, CEWiT, Apple, CMCC, InterDigital, Ericsson, CATT, ZTE, vivo, QC, Xiaomi, Huawei,

- 6 companies supporting for “Indirect connection with AMF involvement” [Nokia, CEWiT, Interdigital, ZTE (optional deployment), vivo (optional), Samsung]

- 4 companies are ok with both options. [CEWiT, InterDigital, ZTE, vivo]

In last meeting, more than 3 companies clearly indicated the objections for supporting both directly and indirectly options. Also considering that 4/6 companies supporting “Indirect connection” are also supporting direct connection option, so it is necessary to further discuss and check the objections on each option and then to decide which option will be selected.

Principle 4a: Sensing control signalling is exchanged between SE and SF directly without AMF involvement.

Supporting: 13?     Objection: ?

Principle 4b: Sensing control signalling is exchanged between SE and SF indirectly via AMF.

Supporting: 6?     Objection: ?

Observation 2b: For Sensing data delivery between SE and SF, the following proposals from companies as:

- 10 companies supporting for “Direct connection without AMF involvement” [OPPO, LGE, CEWiT, Apple, CMCC, Interdigital, Ericsson, QC, Xiaomi, Huawei,

- 2 companies supporting for “Indirect connection with AMF involvement” [CEWiT, Samsung]

As the majority companies preferred, to propose the war forward as Direct connection between SE and SF without AMF involvement.

Proposal 2b: “Principle 5: For Sensing data is delivered directly between SE and SF without AMF involvement.”

(Whether need GSF?)

Observation 3a: for functionality of exposure sensing result to sensing service consumer, 3 companies propose to enhance NEF (Xiaomi, CEWiT, Ericsson). no company proposes GSF.

Proposal 3a: “Principle 6a: NEF is enhanced to support Sensing result exposure to Sensing Service Consumer”.

Observation 3b: for functionality of discovery and selection of SF, 4 companies clearly propose to define new GSF to support for discovery and selection of SF. [OPPO, CATT, QC, ZTE]. But 3 companies (Xiaomi, Ericsson, CEWiT) propose to enhance NEF to support discovery and selection of SF. So, propose to continue to discuss on this issue, and to check the objection of each option.

Proposal 3b, “Principle 6b: new GSF or enhanced NEF is used to support the discovery and selection of SF.”

Observation 2c: for the functionality of authorization for sensing service request, 4 companies propose/implies to define new GSF to support for authorization of sensing service request. but 2 companies propose to use SF to support for authorization of sensing service request. so, propose to continue discuss this issue and to check the objection of each option.

Proposal 3c: “Principle 6c: new GSF or SF is used to support for authorization of sensing service request.”

Observation 3d: for functionality of authorization for Sensing service consumer, if the sensing service consumer is outside the trusted domain, it was already agreed that NEF is enhanced to support such functionality. But for the trusted domain, one company (OPPO) proposes to define new GSF to support authorization for Sensing service consumer, but one company (Xiaomi) proposes to use implementation way to perform the authorization for Sensing service consumer between trusted AF and operator.

Proposal 3d: “Principle 6d: If the AF is inside the trusted domain, it is up to implementation way or defines new GSF to perform the Authorisation of the AF for Sensing Service request”

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * First Change * * * *

8	Conclusions

8.1 conclusion for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

- The functionalities of SCF may support for, e.g., receiving the sensing service request, authorization of the Sensing service request, sending configuration parameters to SE, etc.

- The functionalities of SPF may support for e.g., sensing data collection, sensing result generation, etc.

NOTE 1: the details of functionalities of SCF or SPF will be specified during the normative work.

NOTE 2:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 3:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 4:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

Editor’s Note: whether Sensing control signalling is exchanged between SE and SF directly without AMF involvement is FFS and will be resolved in next meeting.

Principle 5: Sensing data delivery between SE (i.e. gNB) and SF is supported via direct connection without AMF involvement.

Principle 6: Only one SF is selected to support for one sensing service request in this release

NOTE  5:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 6:	Other KI's conclusions will be aligned with KI#1 conclusions.

* * * End of Changes * * * *