3GPP TSG SA WG2 Meeting #172	S2-2510523

Dallas, USA, 17 – 21 November 2025	(revision of)

Source:	Lenovo, Orange

Title:	[WT#3.2 scope update] Support enhanced communication between 3rd party AF and core network

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:.

# 1. Discussion

This contribution provides justification of including within the WT#3.2 scope an objective to study support of communication between 3rd party apps/systems (e.g. AI agents) and the 6G core network.

Such communication is essential for exposing network capabilities and data to AI-driven apps/services to enable them to make more intelligent and context-aware decisions, and to leverage the capabilities offered by the 6G network in an agentic manner.

To enable such communication, this contribution proposes that SA2 studies the following key enablers:

(a) Adoption of industry-standard protocols for AI communication

-	In the AI industry, the Model Context Protocol (MCP) is emerging as a standard for enabling AI apps/services to interact with external tools and systems. MCP provides a common framework that avoids the need for custom-built integrations for each new service.

-	It is proposed to study how the 6G Core Network can leverage industry-standard protocols (such as MCP) to achieve seamless integration with 3rd party AI apps/services (e.g. AI agents). The study should focus on how the CN can offer "tools" that an AI app/service can call upon to securely expose relevant 3GPP network information and capabilities. This would allow AI apps/services to query the network for the data needed to perform their functions more effectively.

(b) Intent-based architecture for network exposure

-	It is also proposed to study the support of an intent-based architecture for network exposure services tailored to AI apps/services.

-	Currently, an application may need to invoke multiple Northbound API requests to gather the necessary data from different network elements. The intent-based approach would simplify this process significantly.

-	Instead of making multiple API requests, a 3rd party AI app/service would send a single, high-level intent to the network (e.g., "provide all necessary network data to optimize the video streaming experience for UE group X"). The Core Network would then be responsible for autonomously interpreting this intent, retrieving the relevant data from the appropriate NFs, and providing a consolidated response to the AI app/service, potentially using the MCP-based communication framework described above.

NOTE: 	This contribution has been partially supported by the HORIZON-JU-SNS-2024 EU project FLECON-6G (GA Number 101192462).

Following updates are proposed to the S2-2509792 that was postponed at SA2#170

We propose to re-word bullet 5 as follows in order to provide more details on the objectives:

-	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s) to support:

-	Study how the 6G Core Network can leverage industry-standard protocols (such as MCP) to achieve seamless integration with 3rd party AI apps/services (e.g. AI agents).

-	Study how to simplify the request for exposure services between a 3rd party AI app/service (e.g. AI agent) and the core network (e.g. introduce an intent-based procedure avoiding the need to invoking multiple northbound API requests to gather the necessary data).

**** First Change****

# Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g., AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.2 6G Network for AI:

1.Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.Study whether and how to enable communication for AI agents on different UEs via the 6G network(s).

3.Support whether and how 6G network supports to authorize and identify the AI Agent(s) on UE.

4.Void

5.Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s)

5.1	Study how the 6G Core Network can leverage industry-standard protocols (such as MCP) to achieve seamless integration with 3rd party AI apps/services (e.g. AI agents).

5.2	Study how to simplify the request for exposure services between a 3rd party AI app/service (e.g. AI agent) and the core network (e.g. introduce an intent-based procedure avoiding the need to invoking multiple northbound API requests to gather the necessary data).

NOTE X: Coordination with WT#1.2 for Network Exposure is required.

NOTE 1: Coordination with SA6 on the application layer aspects and with SA3 on security aspects may be needed.

NOTE 2: When AI agent is on the UE, this AI agent is assumed not part of the MT. It is also assumed that this WT is not replacing  UE to Core Network interaction, NAS protocol.

NOTE 3: The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

6.	Study whether and how the 6G CN can provide AI services (i.e., AI inferencing and AI training) to applications (AF or in UE).

7. Study whether and how the UE/AF can provide AI traffic characteristics to the 6G network so that the network can perform traffic handling (e.g., detection and differentiation) and provide QoS considering the traffic characteristics via user plane.

NOTE 4: Solutions for handling of AI traffic considering the traffic characteristics of AI traffic should be coordinated with WT1.2 for QoS.

NOTE 5: Coordination with SA3, SA5 and SA6 may be needed on AI inferencing and AI training.

NOTE 6: Coordination with WT#5 and WT#6 may be needed.

NOTE 7: New service charging part may be coordinated with SA5 Charging group.

NOTE 8: Coordination with SA1 on the AI traffic characteristics is required.

NOTE 9: Coordination with RAN is required for bullet 7.

**** End of Changes****