3GPP TSG-SA WG2 Meeting #172	S2-2510798

# November 17-21, 2025 Dallas, USA

Source:	Skylo Technologies

Title:	FS_SMS2EC_ARC: Sol#1 updates to resolve EN

Agenda item:	20.8.1

Document for:	Agreement

Work Item / Release: FS_SMS2EC_ARC/Rel-20

Abstract of the contribution: This paper updates Sol#1 to resolve EN

# 1. Discussion

The EN in Sol#1 is addressed in this update.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-65 V0.2.0.

* * * First Change * * * *

## 6.1	Solution #1: Support of SMS to EC indication and SMS user data header for SMS to EC

### 6.1.0	High level principles

The following high-level principles are proposed for the support of SMS to Emergency Centre (EC) service:

-	A header is introduced within the SMS-SUBMIT TPDU, specifically within the TP-UDH. The header may include emergency-related information such as the requested emergency service type, callback identifiers, or other parameters required for accurate routing to a PSAP.

-	When the UE determines that a message is a SMS towards EC message, the UE composes the SMS-SUBMIT TPDU including the header with emergency-related information and submits it using a Service Centre (SC) address configured for handling SMS to EC as outlined in GSMA NG.119 [11]. The SC address is carried in the SMS-SUBMIT RPDU (Relay Protocol Data Unit).

-	The UE may also indicate towards the network at NAS layer that the MO data transport contains a SMS to EC.

-	The serving network forwards the SMS to the SMSC. The SMSC applies emergency-specific processing and delivers the message to the appropriate PSAP, supporting use cases such as Text-to-9-1-1 (see ATIS J-STD-110 [10]).

-	For cases where the UE does not detect that the SMS is a SMS to EC, the message is composed as a normal SMS without header with emergency-related information. The SMSC may inspect the Destination Address (TP-DA) of the SMS. If the TP-DA matches an entry in the network's provisioned emergency-number list, the SMS is classified as a SMS to EC. The network shall then internally flag the SMS as a SMS to EC and ensure that it is routed to the correct PSAP in accordance with applicable regulatory requirements.

### 6.1.1	Description

#### 6.1.1.1	UE Detected and Non UE Detectable SMS to EC

To support reliable detection and routing of SMS to EC, the UE may include a header within the TP-UDH of the SMS-SUBMIT TPDU. The header may carry critical information such as a callback number, emergency service type and location information e.g. pre-registered civic address for the purpose of emergency, GPS/device-based location, or network-based location. Provision of such information at the point of message origination enables the network to more reliably determine whether a MO SMS requires emergency handling and to ensure accurate routing to the appropriate Public Safety Answering Point (PSAP). This mechanism reduces the risk of misrouted or delayed SMS to EC and thereby improves user safety.

The Destination Address TP-DA shall correspond to an emergency short code (e.g. 112, 911). The header in TP-UDH may contain the following emergency-related information:

-	indication that the TP-UD contains a message to EC;

-	callback number, if available;

-	location information (e.g. pre-registered civic address, GPS/device-based location, or network-based location), if available;

-	emergency service type (e.g. police, ambulance, fire).

The serving network may download additional emergency numbers to the UE to enable the UE to detect a SMS to EC.

In the case of non-UE detectable SMS to EC, the message will be handled as a normal SMS. The SMSC shall inspect the TP-DA of the SMS-SUBMIT TPDU. If the TP-DA matches an entry in the provisioned emergency-number list, the message shall be flagged as emergency traffic and routed to the appropriate PSAP in accordance with regulatory requirements.

The SMSC or SMS Router shall inspect the SMS-SUBMIT TPDU. If the header with emergency-related information is present in the TP-UDH, the network shall extract the included information (e.g. callback number, emergency service type and location information) and classify the message as a SMS to EC. The network may query the location retrieval functionalities in the network to obtain authoritative location information, apply local routing policies and ensure that the message is delivered to the correct PSAP in the serving country or region.

### 6.1.2	Procedures

#### 6.1.2.1	Roaming Handling

For emergency messages initiated by a roaming user, the principle of Local Breakout is maintained, prioritizing service from the VPLMN based on local regulations. If the UE does not know its own location, the UE may retrieve its location information from the VPLMN, e.g. LRF lookup and the VPLMN routes the message directly to the local PSAP. The user data header contained in the TPDU maybe used to choose the correct service-type PSAP.

If the VPLMN does not support the required capability to route the MO SMS to the local PSAP, e.g. the VPLMN’s SMSC/SMS Router identify the message as an SMS to EC message (either via the TP-UDH header or TP-DA inspection), but determines it lacks the necessary interconnection or routing capability to deliver the message to the appropriate local PSAP, the VPLMN shall reject the SMS-SUBMIT by returning an appropriate error message to the UE, e.g., an RP-Error message with a specific cause value.

The UE, upon receipt of this rejection, shall handle this by prompting the user to attempt and emergency voice call if available. The network may use CS Fallback (or domain selection) to route the emergency call via the most reliable available domain.

#### 6.1.2.2	Callback Handling

When the PSAP replies, it sends the message back to the TP-OA (MSISDN).

### 6.1.3	Impacts to Services, Entities and Interfaces

UE:

-	Support the inclusion of a header with emergency-related information in the TP-UDH of SMS-SUBMIT when originating SMS to EC.

-	Indicate towards the NAS layer that the MO data transport contains SMS to EC.

SMSC/SMS Router:

-	Inspect SMS-SUBMIT TPDU for presence of the header with emergency-related information in TP-UDH.

* * * End of Changes * * * *