SA WG2 Meeting #172	S2-2510828

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510254)

Source:	ZTE

Title:	pCR to TR 23.700-56: KI#1, New Sol: Early Media in Alerting Phase (CRS) Based on DC AS Initiated A2P DC

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: The paper adds a new solution for supporting early media in alerting phase (CRS) based on DC AS initiated A2P DC in TR 23.700-56.

1. Discussion

The proposal addresses the Key Issue:

Key Issue #1: ADC establishment in alerting phase for early media

This key issue will investigate how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.

This Key Issue will study:

-	Whether and how the capability of supporting ADC establishment in alerting phase is exchanged between UE and IMS network.

-	How the use of ADC during alerting phase is controlled.

-	Whether and how to download a specific list of DC applications that can be used during alerting phase.

-	How to establish and terminate an ADC during alerting phase.

The paper adds a new solution for supporting early media in alerting phase (CRS) based on DC AS initiated A2P DC.

2. Proposal

It is proposed to capture the following changes into TR 23.700-56.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #5 | X |  |  |
| #6 | X |  |  |
| #7 |  | X |  |
| #8 |  | X |  |
| #9 |  | X |  |
| 10 |  |  | X |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |
| #X | X |  |  |

* * * * Next Change (All new text) * * * *

6.1	Solution #X: Early Media in Alerting Phase (CRS) Based on DC AS Initiated A2P DC

6.X.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Ringing Signal (CRS) service which is subscribed by the calling party and it will provide customized content to the called party as an early media during the calling process.

The solution has the following principles:

-	The calling UE A has subscribed to the DC based CRS service.

-	The network serving the calling UE A and the called UE B have the early media capability, and the UE B has negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the calling UE A notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the called UE B.

6.X.1	Description

This solution provides a procedure on how to establish an A2P ADC in alerting phase to provide customized DC content to the called party as an early media for CRS.

6.X.2	Procedure

The pre-condition of the following procedure is that the calling UE A has subscribed to the DC based CRS service and the network serving the calling UE A and the called UE B have the early media capability and the UE B has negotiated with the serving network regarding this capability. It assumes that the DC based CRS service will not coexists with the legacy CRS service; it means the UE will subscribe only one of the CRS services.

Figure 6.X.2-1: Procedure of A2P ADC establishment in alerting phase for CRS

The steps in the procedure are as follows:

1.	UE A initiates SIP INVITE including Audio/Video/BDC SDP; step 1-14 of Figure AC.7.1-1 in TS 23.228 [2] for BDC establishment apply.

2.	BDC has been established for UE B and UE B may download the application list and the application for CRS service.

3.	The IMS AS-1 receives 18X response from the terminating network.

3. Based on the subscription of UE A, IMS AS-1decides to notify the DC AS at the network serving the UE A this BDC establishment event.

NOTE 1:	It assumes that the DC AS has subscribed the BDC establishment event of UE A.

4. The IMS AS-1 sends the Nimsas_ImsEE_Notify Request to the NEF the calling event including the calling UE A and the called UE B information, as well as IMS Session ID.

5.	The NEF sends the Nnef_ImsEE_Notify Request to the DC AS the calling event including the corresponding information.

6.	The IMS AS-1 receives 180 Ringing from the terminating network.

7.	The DC AS decides to establish A2P ADC to UE B to deliver the DC early media for CRS.

8.	The DC AS establishes A2P ADC with UE B; and step 1-21 of Figure AG.2.1.1-1 in TS 23.228 [2] for adding ADC to an existing IMS session apply with the following differences.

-	The A2P ADC is added to an IMS session in establishment phase not an existing IMS session.

-	The SIP signal UPDATE is used instead of re-INVITE in the procedure of AG.2.1.1 in TS 23.228 [2].

UE B may download the application used in the alert phase.

The DC AS may send the A2P ADC addition request to the IMS AS-1 prior to the 180 Ringing. The IMS AS-1 will hold the request until it receives the 180 Ringing from the UE B.

9.	The UE B/terminating network and UE A/originating network exchange 180 RingingSIP signals.

10.	The A2P ADC between UE B and the DC AS which goes through the MF is activated and the early media (customized alerting tones) of this IMS session is delivered from the DC AS to UE B. The user of UE B experiences with the early media for CRS.

11.	The user of UE B picks up the phone and the UE B/terminating network and UE A/originating network exchange 200 OK (INVITE) SIP signal.

12.	The IMS AS-2 notifies the DC AS at the calling network the IMS session establishment event.

13.	The DC AS releases the A2P ADC with UE B.

14.	Subsequent procedures continue.

6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting ADC establishment in alerting phase for CRS.

IMS AS:

-	supporting report of IMS session establishment events to DC AS;

-	supporting A2P DC establishment for delivering early media.

MF:

-	receiving early media from DC AS and providing it to UE through A2P DC.

DC AS:

-	supporting receiving IMS session establishment events from IMS AS through NEF;

-	supporting initiating and releasing A2P DC for delivering early media.

* * * * End of Changes * * * *