3GPP TSG-SA WG2 Meeting #172	S2-2510921

17-21 November, 2025, Dallas , US 	(Revision of S2-2510827,10357)

Source:	China Mobile

Title:	WT#1: Update solution for Supporting dynamic configuration of MDC2 endpoints

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes update to solution #12 for KI#3 of TR 23.700-56.

1. Discussion

This contribution proposes update to solution #12 to enhance DC3 and DC4 to support dynamic configuration of MDC2 endpoints when a P2A or P2A2P DC are established.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

6.12	Solution #12: Support dynamic configuration of MDC2 endpoints with new DCSF services

6.12.0	High-level solution Principles

This solution addresses KI#3: Supporting dynamic configuration of MDC2 endpoints.

This solution takes the existing Rel-18 procedures of P2A and P2A2P DC establishment as the basis and updates the existing steps between DCSF and DC AS by defining new service of DCSF.

6.12.1	Description

This solution defines a new service of DCSF which is used to request to establish, update or delete a MDC2 connection. This service exchanges the MDC2 media points information on MF and DC AS to each other. The connection ID is allocated by DCSF and is used to correlate the MDC2 connection.

The interaction between DCSF and DC AS is designed as event exposure of MDC2 connection request to exchange the MDC2 media endpoint information. By this principle, the DC AS from third party customer is not required to provide any service when using IMS DC services from the operator. Accordingly the DCSF services use Subscription/Notification mode. In this solution implicit subscription is used based on the following consideration:

-	If explicit subscription is used, it is very difficult to ensure the DCSF selected by the DC AS to subscribe is the same DCSF selected by the IMS AS during DC establishment, especially when multiple DCSFs are deployed in a pool;

-	Explicit subscription saves cost interaction.

The solution supports both P2A and P2A2P DC management. When P2A DC is established, the media information of one pair of MDC2 media points between MF and DC AS is exchanged. When P2A2P DC is established, the media information of two pairs of MDC2 media points for originating side and terminating side between MF and DC AS is exchanged.

This solution uses event notification and notification response messages to exchange MDC2 media endpoints information. The reliable transmission of the messages is ensured by the transportation layer, e.g. TCP or SCTP.

To support implicit subscription and notification, the solution also clarifies that the DCSF selects the correct DC AS and get the address of the selected DC AS URL based on binding information which indicates which DC AS is serving the application. The mapping of the app id and the DC AS URL is locally configured on DCSF or is retrieved from the application data in the HSS. This mechanism has been specified in Rel-19 to enable the DCSF to fetch Avatar ID list from a DC AS (see clause AC.11.1 of TS 23.228).

6.12.2	Procedures

6.12.2.1	P2A DC procedure

This procedure depicts the procedure of UE initiated P2A DC establishment, P2A DC modification and P2A DC deletion, which reuses the P2A DC establishment procedure in clause AC.7.2.2 of TS 23.228 as the basis and updates original step 9 and 10 to clarify the interaction between DCSF and DC AS.

Figure 6.12.2.1-1 Procedure of P2A DC establishment

The steps in the procedure are as follows:

Step 0-8 is the same with step 0-8 in clause AC.7.2.2 of TS 23.228;

- The DCSF selects the DC AS serving this session based on the binding information of this P2A DC. The DCSF gets the DC AS URL of the selected DC AS by local configuration or retrieves it from DCSF service specific data stored in HSS.

- The DCSF invokes Ndcsf_MDC2Connection_Create service operation as defined in clause 6.X.3.2 to request the selected DC AS to establish MDC2 connection with the MF for the P2A DC. The service operation includes the MDC2 media endpoint information on the MF.

- The DC AS responds to Ndcsf_MDC2Connection_Create service operation with connection ID and its MDC2 media endpoint information for the MDC2 connection.

- 12-28. The same with step 11-27 in clause AC.7.2.2 of TS 23.228.

- 29-30. The UE initiates IMS session modification to update the established P2A DC. The MDC2 media endpoint on the MF is updated and the information is returned to the DCSF.

- 31. The DCSF invokes Ndcsf_MDC2Connection_Update service operation as defined in clause 6.X.3.3 to notify the DC AS the media endpoint on the MF for the established P2A DC is updated. The service operation includes the connection ID and the updated MDC2 media endpoint information on the MF.

- 32. The DC AS updates its media endpoint accordingly and responds to Ndcsf_MDC2Connection_Update service operation with updated MDC2 media endpoint information.

- 33-34. The IMS session is updated.

- 35. The UE releases the P2A DC or the whole IMS session.

- 36-37. The DCSF notifies the DC AS the P2A DC is deleted. The DC AS removes its media resource of the P2A DC.

6.12.2.2	P2A2P DC procedure

This procedure depicts the procedure of UE initiated P2A2P DC establishment, P2A2P DC modification and P2A2P DC deletion, which reuses the P2A2P DC establishment procedure in clause AC.7.2.3 of TS 23.228 as the basis and updates original step 9 and 10 to clarify the interaction between DCSF and DC AS.

Figure 6.12.2.1-1 Procedure of P2A2P DC establishment

The steps in the procedure are as follows:

Step 0-8 is the same with step 0-8 in clause AC.7.2.3 of TS 23.228;

- The DCSF determines to establish P2A2P DC and invokes Ndcsf_MDC2Connection_Create service operation to request the DC AS to establish MDC2 connection with the MF for the P2A2P DC. The service operation includes the MDC2 media endpoint information for originating side and terminating side on the MF.

- The DC AS responds to Ndcsf_MDC2Connection_Create service operation with connection ID and its MDC2 media endpoint information for originating side and terminating side.

- 11-26. The same with step 11-27 in clause AC.7.2.3 of TS 23.228.

- 27-28. The UE initiates IMS session modification to update the established P2A2P DC. The MDC2 media endpoint on the MF is updated and the MDC2 information of originating side is returned to the DCSF.

- 29. The DCSF invokes Ndcsf_MDC2Connection_Update service operation as defined in clause 6.X.3.3 to notify the DC AS the media endpoint for originating side on the MF for the established P2A2P DC is updated. The service operation includes the connection ID and the updated MDC2 media endpoint information on the MF.

- 30. The DC AS updates its media endpoint accordingly and responds to Ndcsf_MDC2Connection_Update service operation with updated MDC2 media endpoint information.

- 31. The IMS session is updated.

- 32. The UE releases the P2A DC or the whole IMS session.

- 33-34. The DCSF notifies the DC AS the P2A DC is deleted. The DC AS removes its media resource of the P2A DC.

6.12.3	DCSF Services

This solution defines DCSF services to support interaction between DCSF and DC AS for establishing the P2A or P2A2P DC.

Table 6.12.3.1-1: DCSF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_MDC2Connection | CreateNotify | Subscription/Notification | DC AS |
|  | UpdateNotify | Subscription/Notification | DC AS |
|  | DeleteNotify | Subscription/Notification | DC AS |

6.12.3.1	Ndcsf_MDC2Connection

#### 6.12.3.1.1	General

Service description: This service enables the consumer to be notified of events of establishing, updating or deleting MDC2 connection towards the MDC2 endpoint on the MF.

#### 6.12.3.1.2	Ndcsf_MDC2Connection_CreateNotify service operation

Service operation name: Ndcsf_MDC2Connection_CreateNotify

Description: This service operation enables DCSF to nofity the consumer of the event of establishing a MDC2 connection with the MF towards the MDC2 endpoint on the MF and responds with its own MDC2 endpoint information on the MF.

Inputs, Required: Session ID, Connection ID, DC type, remote MDC2 media endpoint information

Session ID is the identity of the IMS session to which the MDC2 connection belongs.

Connection ID is the identity of the MDC2 connection. It is allocated by the DCSF.

DC type indicates the type of the established DC, i.e. P2A, P2A2P.

Remote MDC2 media endpoint information contains the following:

-	Endpoint type: Originating side, Terminating side;

-	the IP address and port of the MDC2 media endpoint on MF.

NOTE:	If the DC is P2A DC, only media endpoint information of originating side is included.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. The Calling ID and Called ID can be used by DC AS to decide whether a P2A2P DC should be further established.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

Local MDC2 media endpoint information contains the following:

-	Endpoint type: Originating side, Terminating side;

-	the IP address and port of the MDC2 media endpoint on MF.

NOTE:	If the DC is P2A DC, only media endpoint information of originating side is included.

#### 6.12.3.1.3	Ndcsf_MDC2Connection_UpdateNotify service operation

Service operation name: Ndcsf_MDC2Connection_UpdateNotify

Description: This service operation enables DCSF to notify the consumer the event of updating an existing MDC2 connection on the MF.

Inputs, Required: Session ID, Connection ID, updated remote MDC2 media endpoint information.

Inputs, Optional: None.

Outputs, Required: Result indication,.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

#### 6.12.3.1.4	Ndcsf_MDC2Connection_DeleteNotify service operation

Service operation name: Ndcsf_MDC2Connection_DeleteNotify

Description: This service operation enables DCSF to notify the consumer the event of deleting an existing MDC2 connection on the MF.

Inputs, Required: Session ID, Connection ID.

Inputs, Optional: None.

Outputs, Required: Result indication,.

Outputs, Optional: None.

# * * * * End of changes * * *