3GPP TSG-SA WG2 Meeting #172	S2-2510922

17 - 21 November, 2025, Dallas, USA                             (was 10859, 9916)

Source: 	vivo, Qualcomm Incorporated, China Mobile, MediaTek Inc., CATT, China Unicom, Spreadtrum, NTT DOCOMO, Xiaomi, OPPO, SHARP, Thales, EchoStar, CSCN, TNO, Deutsche Telekom, NEC, China Telecom, LG Electronics

Title: 	Conclusion for KI#1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an updated interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC on IMS voice data transport.

1	Discussion

The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-  The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

In SA2#171 meeting, the interim conclusion has been agreed as following (see S2-2509293):

Based on above key interim conclusion for IMS voice data and the solutions on the table, further interim conclusions need to be provided for following key points on KI#1:

- IMS voice data over IP vs non-IP

The discussion paper S2-2509915 has provided a detailed technical analysis and evaluation on above key point with following observations:

- Observation 1: In the high possibility cases, UP IP and UP non-IP has almost the same level of protocol overhead (UP IP: 1-3B, UP non-IP: 2B) when using ROHC in fully compressed state.

- Observation 2: UP IP has less UE and network impacts while the key challenge is performance of ROHC stability; UP non-IP has bigger UE and network impacts while there are no performance stability risks; Both UP IP and non-IP have the same RAN impacts.

- Observation 3: LS response from RAN1 indicated that ROHC performance stability can be considered guaranteed for LOS scenarios. However for NLOS scenarios, ROHC performance stability cannot be guaranteed.

- Observation 4: The configuration & implementation of ROHC functions are out of RAN2 scope as well as SA2 scope. 3GPP has no control of ROHC based solutions during the commercial deployment.

- Observation 5: Non-IP option has standard impacts related to protocol conversion.

Based on above observations, following proposal is provided:

- Proposal: For IMS voice data transfer, conclusions for both IP and non-IP solutions should be captured in TR for the normative work.

2 Proposal

It is proposed to capture the following contents into TR 23.700-19 v1.1.0.

# * * * * First Change* * *

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	For the transport mechanism of voice packets over the NB-IoT (GEO) user plane:

a)	Transport mechanism of IP packets shall be supported, use of Robust Header Compression (RoHC) is recommended for this case; and

b)	Transport mechanism of voice packets using removal and restoration of parts of RTP/UDP/IP headers in the UE and network can be supported.

Editor's Note: Where the RTP/UDP/IP headers are removed and restored e.g. in PGW or IMS is FFS.

# * * * * End of change * * * *