SA WG2 Meeting #172	                              S2-2510897

17 - 21 Nov, 2025, Dallas, USA

Source:	China Mobile, ZTE, SK Telecom, Rakuten Mobile, Nokia, CATT, vivo, Samsung, AT&T, Deutsche Telekom, Verizon, ETRI, Huawei, HiSilicon, Ericsson

Title:	Update for KI#2 Interim Agreements and Conclusions for UC1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for AIML_CN KI#2.

- Discussion

In TR 23.700-04 V0.3.1 (2025-09), some principles are agreed for KI#2. However, there are still several topics for further consideration depicted as 7.2.2 in TR 23.700-04.

The following topics are for further consideration for KI#2 for UC1:

-	The need and details of type/identification of traffic pattern is FFS.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-  For UC1, the input from AF, possibily via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

NOTE : How UPF subscribes to the new analytics is FFS.

This proposal discusses these topics and concludes the aspects for KI #2 for UC1.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** First change ***

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288) for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations

- UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations)

-	adjust packet processing resources,

- NOTE 1: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

*** Second change ***

8	Conclusions

Regarding KI #2, the following principles are concluded for UC#1:

-	A new Analytics ID “Abnormal user plane traffic” will be introduced for UC#1. The consumer may be SMF, PCF, UPF and AF.

NOTE 1: Whether OAM can be the Analytics consumer will be determined in the normative phase, coordination with SA5 is needed.

-	The output analytics, both statistics and/or predictions, include at least:

-  Identification of the traffic flow (e.g. traffic flow descriptors). For each identified traffic flow:

-	Type of abnormal traffic.

-  Information of the detected anomality, such as (e.g. malformed, unexpected traffic volume, burst, etc.), malicious (e.g. DDoS).

-	Volume, rate or burst size of abnormal traffic.

-	Identifiers/addresses of UPF(s), interface if applicable, list of UE(s), list of PDU session(s) affected by identified abnormal traffic type.

-	Identifiers/ information of the traffic source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	Time window (e.g. start, duration).

-	To derive the output analytics, the following input data may be collected from the UPF, SMF or AF. (If individual UEs are targeted, the NWDAF subscribes at the SMF to obtain related information from the UPF)

-  Information identifying the associated application/service data flow(s) (e.g. packet filter, the traffic flow, Source and/or, destination IP address and port, Protocol Type, URL lists, QFI, Application ID).

-	Identifiers of corresponding UEs.

- 	Information of traffic characteristics from SMF or UPF, e.g. measured UL/DL data volumes, measured UL/DL data rates.

-	Information of UP pattern, e.g. pattern type, including malformed, unknown, duplicate, fragmented, etc.

NOTE 3: Whether any additional traffic identifier is needed will be determined during normative work.

-  Traffic characteristics of normal traffic and abnormal traffic from external server/AF, and the abnormal type for the abnormal traffic, which can be used by the NWDAF to determine whether the input traffic is abnormal traffic by comparing the characteristics of input traffic with the characteristics of normal traffic or abnormal traffic.

The event subscription by NWDAF to UPF can Target any UE or specific UEs and provides information about targeted traffic abnormalities, thresholds for the volume/burst and/or sampling periods. When the NWDAF requests to report traffic abnormalities, then the UPF only reports when corresponding traffic abnormalities are detected and threshold are exceeded.

NOTE 4: How UPF provides information of traffic abnormalities to NWDAF will be determined in the normative phase.

NOTE 5:  Related UPF and SMF events will be determined in the normative phase. Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined will be determined in the normative phase.

The SMF as consumer of NWDAF analytics may determine the mitigation actions for example:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501 [2] and clause 4.3.5 of TS 23.502 [3]).

-	Configuring UPF to enforce the downlink traffic suppression (e.g., selective packet dropping or packet per second limitations) or shape abnormal traffic, to enforce bandwidth limitations. For each observed traffic anomaly that can be reported by the NWDAF, corresponding mitigation actions are configured in the SMF. Then, the SMF provides them to the UPF. The SMF may use actions on non-PDU session level applicable to any PDU session.

The UPF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	Downlink traffic suppression (e.g. selective packet dropping or rate limitations).

-  When the UPF is consumer, the subscription initiation follows the “Subscribe-Notify” as per TS 23.501 [2] clause 7.1.2 (Figure 7.1.2-3). In this case, the SMF subscribes to NWDAF services on the new Analytics ID on abnormal data packets patterns on behalf of the UPF and indicate that reduced output is requested, and the NWDAF can send directly the analytics to the UPF for activation and deactivation of the related rules in the UPF for the abnormal data packet handling, such as enforcing the downlink traffic suppression (e.g., selective packet dropping or packet per second limitations) or adjust the packet processing resources.

NOTE 6: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor).

NOTE 7: Whether and how to configure the UPF with actions from OAM or SMF for abnormal traffic handling will be determined during normative work.

As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

The PCF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-  Policy creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting) or adjusting QoS parameters.

The AF (e.g. IoT server) as consumer of NWDAF analytics could take action to correct abnormal traffic.

*** End of the changes ***