SA WG2 Meeting #171	S2-2511011

Dallas, USA, 17 – 21 November 2025                  	(revision of S2-2510984)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0106 | rev | 8 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Inventory procedure for use of Temporary Device ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc., Sony, Ericsson, Huawei, Lenovo, FutureWei, Samsung, ZTE, CATT, Oppo, NTT DOCOMO, NEC, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 has recently agreed on the mechanism of using Temporary Device ID in Inventory procedure and it is specified in the latest TS33.369. Inventory procedures in TS 23.369 needs to be aligned with TS 33.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Inventory procedure to align with that in TS 33.369. In rev8, the following changes are added: 1) clarify triggers for inventory all in step 1. 2) term alignment on AIoT Identification Information. 3) reference to the network performing procedure for the resynchronization of T-ID specified in TS 33.369. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Inventory procedure in TS 23.369 is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command. Clause 6.2.5 provides the AIoT Session release procedure for AIoT Inventory and AIoT Command.

The AIOTF may perform the procedure in Clause 6.2.2 or Clause 6.2.3 for the resynchronization of T-ID as specified in clause 5.4.4 of TS 33.369 [9].

SECOND CHANGE

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory Request (AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) service operation request to the NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s). If Information about the target AIoT Device(s) is not included, the Inventory Request is for all AIoT Devices.

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The time interval, if provided, is described in clause 5.9.

The location information requested parameter indicates whether the AF expects the network to provide the device location information.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the Nnef_AIoT_Inventory request with an appropriate cause code and step 6 is performed before ending the procedure

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], [location information requested]) service operation towards each of the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Inventory request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the AIOTF request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF obtains security parameters from the ADM, as described in TS 33.369 [9].

The AIoT Identification Information to be provided to NG-RAN contains either Filtering Information as defined in clause 5.8, an AIoT Device Permanent Identifier as defined in clause 5.7.2 or an AIoT Device Temporary Identifier as defined in clause 5.7.x.

When the AIoT Identification Information includes an AIoT Device Temporary Identifier, the AIoT Device Temporary Identifier and related information is determined as specified in TS 33.369 [9].

If no AIoT Identification Information is provided to NG-RAN, then the NG-RAN inventories all AIoT Devices.

NOTE:	The concerns of paging all or paging a large group of AIoT devices are described in TS 33.369 [9].

The AIoT Identification Information and security parameters are provided to NG-RAN in step 7.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF may perform AMF selection as described in clause 5.3.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including AIoT Identification Information and security parameters determined in step 4 to be included in the paging message, Correlation ID, Requested Service Area Information, and assistance information to the selected NG-RAN as specified in TS 38.413 [10].

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the AIoT service operation accordingly as specified in TS 38.413 [10]. If the Inventory Request is not rejected, then an AIoT Session is created in the NG-RAN. After this step, the AIoT Session between the NG-RAN and AIOTF identified by correlation ID is established.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300 [5] and TS 38.391 [11]. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information and the security parameters.

If the received AIoT Identification Information contains:

-	Filtering Information, the AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

-	AIoT Device Temporary Identifier, the AIoT Device determines whether it matches the AIoT Identification Information, as described TS 33.369 [9].

-	AIoT Device Permanent Identifier, the AIoT Device determines whether it matches the AIoT Identification Information by comparing it with the stored AIoT Device Permanent Identifier.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its device specific authentication information as specified in TS 33.369 [9] and optionally the AIoT Device Permanent Identifier if privacy protection is not used.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [10]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.9. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF authenticates the AIoT Device, and retrieves the AIoT Device Permanent Identifier as specified in TS 33.369 [9]. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete and the last inventory result. After the procedure has completed NG-RAN will not send any further Inventory Reports for this requested Inventory.

Editor's note:	The details about completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the Naiotf_AIoT_Inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s) and optionally location of each AIoT Device. The AIOTF may send multiple reports.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN in step 10 to determine the AIoT Device Location.

The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. When the last report is sent, the AIOTF ends the AIoT Session. If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Notify's from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the Nnef_AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s) and optionally location of each AIoT Device. The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

END OF CHANGES