SA WG2 Meeting #172	S2-2511254

Dallas, USA, 17 - 21 November, 2025                                   	  (was S2-2511164)

Source:	CATT, Ericsson, OPPO, LGE, NTT DOCOMO, Skylo Technologies, NEC, Novamint, Thales, TNO, EchoStar, Inmarsat, Viasat,  Sateliot, Airbus,  Eutelsat, Vivo, Spreadtrum, CSCN, MediaTek Inc., China Unicom, China Telecom, China Mobile, ZTE corporation, Xiaomi, QC, Nokia

Title:	[WT#7]: Work task and key issue for 6G satellite access

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the scope of WT#7 and corresponding Key Issue(s) for SA2 6G SID.

1. Justifications

SA2 SID S2-2506096 proposes to study the aspects of non-terrestrial networks (NTN) in 6G. NTN topic is included in the following WTs:

-	WT#2: Study migration and interworking, including

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

-	WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

In addition, 3GPP RAN groups also agree to include NTN in the 6G Radio SI (see RP-251881) and will design harmonized 6G Radio for TN and NTN.

2. Merged Documents

The following contributions are considered to the WT#7: 6G NTN for SA2#172 meeting.

| TDoc | Source | WT | KI |
| --- | --- | --- | --- |
| S2-2509940 | NEC | Same as S2-2509796 | 5.X. Key Issue #Y(7.1): Support 6G RAT for NTN in 6G system This key issue will study how to support satellite access using 6G RAT for NTN in 6G system, considering transparent and regenerative payload architectures. The key issue will also study how to include support for: - Store and Forward (S&F) mode. - Policy/charging differentiation and (re)selection among TN and satellite accesses with different orbit types (LEO, MEO, GEO). - Lawful Interception. NOTE 1: The detailed scope on supporting 6G RAT for NTN will be coordinated and aligned with RAN. NOTE 2: Whether and how to re-use or enhance the 4G/5G satellite specific functions(e.g. earth fixed tracking areas, satellite specific RAT types etc.) should be evaluated. NOTE 3: The detailed scope of Lawful Interception will be coordinated and aligned with SA3-LI. NOTE 4: The deployment of CN functionalities on-board a satellite should support the optimization of the UP path (e.g. by the Inter-satellite link) and take the deployment complexity into consideration. NOTE 5: The mechanism design to tackle intermittent connectivity via NTN should follow the general principle designed to tackle intermittent connectivity via TN. NOTE 6: The policy/charging including the (re)selection among TN and satellite accesses with different orbit should be coordinated and aligned with general policy/charging framework in WT#1.2 and CT1/RAN2’s conclusions. 5.X. Key Issue #Z(7.2): Mobility and service continuity This key issue will study how to support mobility and service continuity for NTN due to user or satellite mobility or other network conditions, including: - Whether and how to enhance mobility and service continuity between TN and NTN, including support for simultaneous connectivity via TN and NTN (intra- and inter-operator cases). - Whether and how to enhance mobility and service continuity between access links via satellitesin the same and different orbit types (LEO, MEO, GEO). - Enhanced reliable connectivity via multiple 6G NTN access links (from different orbits) to support satellite backhaul. - How to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance. 5.X. Key Issue #M(7.3): Interworking with legacy system/RAT This key issue will study whether and how to support interworking for NTN with legacy system/RAT, especially the following aspects: - Whether and how to support interworking with legacy system/RAT. NOTE: How to handle interworking with legacy system/RAT will be coordinated and aligned with key issues related to WT#2 |
| S2-2510238 | Ericsson | Based on S2-2509796, modified: NOTE 3: This WT is dependent on overall architecture and mobility aspects that are scoped under WT#1 (including sub-WTs). | It is proposed to copy WT text as KI |
| S2-2510281 | Apple | Based on S2-2509796, added: NOTE 4: This WT has NTN specific aspects for voice that are scoped under WT#1.4. | - How to support satellite access in the 6G system. NOTE 1: Further works topics may be introduced based on RAN progress on 6G NTN. NOTE 2: This KI has IWK aspects that are scoped under WT#2. NOTE 3: This KI has mobility aspects that are scoped under WT#1.1. NOTE 4: This KI has NTN specific aspects for voice that are scoped under WT#1.4. Editor's Notes: Which satellite specific functions defined in 5G system will be reused in 6G system is FFS. Editor's Notes: Whether and how to re-use 4G satellite specific functions, such as store and forward, is FFS. |
| S2-2510442 | Xiaomi | Same as S2-2509796 | 1) How to support 6G RAT for NTN in the 6G system. 2) How to support service continuity when UE using 6G NTN. NOTE 1: The detailed scope for WT#7 will be coordinated and aligned with RAN, including multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions. NOTE 2: This WT has interworking aspects that are scoped under WT#2. NOTE 3: This WT has mobility aspects that are scoped under WT#1.1. |
| S2-2510465 | MediaTek Inc. | Same as S2-2509796 | For the support of 6G NTN, following aspects shall be considered and studied: - How to support satellite access in the 6G system. - How to support session continuity for satellite access in the 6G system. NOTE 1: Further works may be introduced based on RAN progress on 6G NTN. Editor's Notes: Which satellite specific functions defined in 5G system will be reused in 6G system is FFS. Editor's Notes: Whether and how to re-use 4G satellite specific functions, such as store and forward, is FFS. |
| S2-2510471 | CSCN | Based on S2-2509796, added: NOTE 4: This WT has QoS aspects for satellite backhaul that are scoped under WT#1.2 for QoS. | NA |
| S2-2510566 | Huawei, HiSilicon, TNO | Based on S2-2509796, added: - Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance. | copy the WT. |
| S2-2510579 | NTT DOCOMO | Same as S2-2509796 (with a few wording changes according to offline discussion) | This key issue for the support of NTN will study the following: - Study service continuity aspects for NTN, including TN–NTN and NTN–NTN mobility. - Perform a high-level gap analysis to determine whether existing mobility and continuity mechanisms in the 6G architecture are sufficient for multi-orbit and dynamic NTN cases. - Identify architectural dependencies and coordination points between RAN and CN to ensure coherent NTN support. - How to ensure session continuity without packet loss during orbit transitions in case of multi-orbit architecture, while satisfying those service-specific requirements. - Study how to support path selection and routing coordination for NTN, including scenarios where multiple satellite orbits are available, for maintaining service continuity. - How to ensure interworking between the multi-orbit NTN system and other accesses, including 3GPP accesses in the terrestrial network and non-3GPP satellite accesses. |
| S2-2510610 | CATT | Same as S2-2509796 (with a few wording changes according to offline discussion) | For the support of 6G NTN, following aspects shall be considered and studied: - How to support satellite access in the 6G system. - Whether and how to ensure session/service continuity across different satellite access networks, or between terrestrial and satellite access networks in 6G system. - How to efficiently support RAN node mobility (in regenerative payload mode). - Whether and how to improve QoS/policy for satellite access and satellite backhaul. |
| S2-2510637 | ZTE | Based on S2-2509796, modified: NOTE 1: Based on RAN decision, WT/KI descriptions may need to be revisited before discussing solutions. | NA |

3. Text proposal

It is proposed to agree the following changes to TR 23.801-01:

**** First Change (all text new)****

A.X	Work Task 7: : Support of 6G NTN

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has dependency on mobility aspects that are scoped under WT#1.1.

NOTE 4:	This WT has voice aspects that are scoped under WT#1.4.

**** Second Change (all text new) ****

5.X. Key Issue #X: Support of 6G NTN

Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This KI has IWK aspects that are scoped under WT#2.

NOTE 3:	This KI has dependency on mobility aspects that are scoped under WT#1.1.

NOTE 4:	This KI has voice aspects that are scoped under WT#1.4.

**** End of Changes ****