3GPP TSG-SA WG2 Meeting #172	S2-2511305

Dallas, USA, 17 – 21 November 2025             (revision of S2-2510947, 10862, 10379)

Source: 	ZTE, CMCC, vivo, China Telecom, MediaTek Inc, Sanechips, LG Electronics, NTT DOCOMO, CALTTA

Title: 	Interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC regarding the evaluation of different PDN connection types and other technical parts.

1	Discussion

In TR 23.700‑19, a total of thirteen candidate solutions are identified to support IMS voice calls over NB‑IoT NTN via GEO satellite connecting to the EPC, covering different numbers of PDN connections (single or dual), either IP‑type or Non‑IP‑type PDN connections, and either pre‑established or on‑demand established dedicated bearers.

For the number of PDN connection:

- The current IMS architecture typically uses a single PDN connection with a common APN, where signaling and voice media are transmitted over separate EPS bearers.

- If two separate PDN connections are used to support IMS signaling and voice, each session requires an independent address, session context, QoS flow, policy control procedures, and resource allocation. This significantly increases the state maintenance burden on both the network side (e.g., P-GW, PCRF) and the UE.

- When signaling and media are associated with different IP addresses, the network must handle the mapping between them, which may cause some issues. For example, during call setup procedure, if the signaling and media IP addresses are not aligned, the session binding maybe failed.

Proposal 1: A single PDN connection shall be used to transport both IMS signalling and IMS voice.

For the type of PDN connection:

- IMS services rely on the SIP/UDP/IP protocol for signaling exchange, while media streams are typically transported over RTP/UDP/IP. Both signaling and media require an IP address to establish an end-to-end transport path.

- If the PDN connection type is non-IP, the IMS client in the UE will be significantly impacted due to the absence of a UE IP address.

Proposal 2: The PDN connection type for IMS voice call over GEO shall be IP.

For the timing of dedicated bearer establishment:

- Based on S2‑2508341, multiple approaches to enhance IMS—including IP or non‑IP voice transmission, text‑based or binary‑based signaling simplification, and deployment with or without B2BUA—were evaluated. The measured call setup time (CST) in these cases ranges from 9.3 s to 15 s, all assuming on‑demand dedicated bearer establishment. Establishing the bearer involves three messages between the UE and RAN (two uplink, one downlink), and the estimated EPS bearer setup delay is about 2.5 s, accounting for approximately 17–27% of the total CST. Therefore, pre‑establishing the dedicated bearer during the Attach/PDN connection setup can significantly reduce the overall call setup time.

- Pre‑established dedicated bearer during attach or PDN connection establishment procedure does not occupy radio resources continuously, as resource utilization depends on actual data transmission and network scheduling. In the absence of IMS voice traffic, no Uu interface resources (e.g., PRBs) will be consumed at all. What’s more, under GEO scenario, for the majority time, the UE should stay in idle mode because the usable applications are miserably few due to limited data rate. During this time, the RRC connection and associated radio bearers are fully released. Upon re‑entering connected mode, the dedicated bearer is re‑established without causing any increase in call setup time.

- When the dedicated bearer is established only upon a user initiating an MO IMS call, its activation must be triggered by the IMS application through a cross‑layer mechanism within the UE—from the IMS client down to the NAS layer. In addition, this approach provides no optimization for MT calls, as the UE cannot anticipate incoming IMS sessions, and the dedicated bearer can only be established on demand.

Proposal 3: The dedicated bearer can be pre-established during attach or PDN connection establishment procedure.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

This pCR is to be implemented after S2-2511304.

# * * * * First Change* * *

8.x		Conclusions for Key Issue #1

Editor's note:	Other conclusions for Key Issue#1 are FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	A single PDN connection shall be used to transport both IMS signalling and IMS voice.

# * * * * End of Changes* * *