3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-211091-r2

- e-meeting, 1 - 5 March 2021											Merge of S3-210958 and S3-211091

- Source:	Ericsson, Huawei, Hisilicon

- Title:	AMF reallocation: Update to solution #2

- Document for:	Approval

Agenda Item:	2.17

# 1	Decision/action requested

This contribution proposes an update to solution #2 for AMF reallocation procedure when 5G NAS security context is rerouted via RAN.

# 2	References

[1]	3GPP TR 33.864 "Study on the security of Access and Mobility Management Function (AMF) re-allocation".

[2]	3GPP TS 33.501 "Security architecture and procedures for 5G system".

[3]	3GPP TS 23.502 "Procedures for the 5G System (5GS)"

[4]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) "

# 3	Rationale

The following updates are proposed in this contribution:

1.	Resolve editor note:

Editor's Note: The security impact of exposing Kamf to RAN is FFS.

In this solution the Kamf-1 key is exposed to RAN. Such exposure of NAS keys to RAN has not been done before and has risks. However, the risk seems to be low because of the following design:

A.	The RAN still is a trusted network entity.

B.	The Kamf-0 key is never exposed to RAN. Due to performing horizontal Kamf derivation of the Kamf-0 key in the initial AMF and only providing the new horizontally derived Kamf-1 to the RAN, the new generated Kamf-1 key could be seen as a one-time key for the purpose of the AMF re-allocation. It is worth noting that Kamf-1 is practically useless to a legitimate RAN node since it is a NAS key that has not been put into use by any AMF in the network.

C.	The target AMF needs to take the new generated Kamf-1 key received from RAN into use with the UE by running a NAS SMC procedure prior to initiating a primary authentication immediately thereafter in order to replace the Kamf-1 key received from the RAN with a new Kamf-2 key. The Kamf-1 key is invalidated as soon as the Kamf-2 key is activated after a new primary authentication. So, the Kamf-1 key becomes immediately useless.

D.	Before the new primary authentication procedure is initiated in order to replace the Kamf-1 key with a new Kamf-2 key, there are risks as an attacker in RAN which has access to Kamf-1 key could misuse the key and perform attacks on the UE and the target AMF. As the target AMF can determine that a NAS re-route via RAN has taken place, the target AMF can restrictively use the Kamf-1 only for the purpose of sending protected NAS Security Mode Command and Authentication Challenge/Request to the UE, and for receiving protected NAS Security Mode Complete and Authentication Response from the UE.

As the risks for new attacks seems to be very low, this solution should be acceptable for the sake of availability to handle the corner-case of re-route via RAN. The evaluation clause has been updated to reflect these arguments.

2.	Resolve editor note:

Editor's Note: The requirement and processing of Reroute NAS response at step 14 is FFS.

It is proposed to delete step 14 in solution #2 as the corresponding step is not described in TS 23.502 [2].

3.	Steps 3a and steps 3b has been further clarified for:

-	These steps do not take place in the case a SUCI is included in a Registration Request message.

-	These steps do not take place in the case a 5G-GUTI is included in a Registration Request message but there is no N14 interface between the initial AMF and the old AMF. in this case the initial AMF requests the UE identity SUCI from the UE in step 4 and then initiate primary authentication in step 5

-	These steps take place in the case a 5G-GUTI is included in a Registration Request message and there is N14 interface between the initial AMF and the old AMF.

4.	Step 12 has been clarified that if the Initial AMF performs horizontal Kamf derivation then the initial AMF resets the corresponding uplink and downlink NAS COUNTs.

5.	Step 13 has been clarified that the Initial AMF forwards the uplink/downlink NAS COUNTs included in the 5G NAS security context to the RAN.

6.	Step 15 has been added and has been further clarified that:

-	After receiving the Registration Request, if SUCI is included, the target AMF skips step 15 (as no additional information about established PDU sessions etc. is stored in the old AMF).

-	If a 5G-GUTI is included in the Registration Request and the target AMF has received a 5G NAS security context and potentially a keyAmfHDerivationInd indicator, then:

a.	If there is no connectivity between the target AMF and old AMF, the target AMF skips step 15 (and any additional information about established PDU sessions etc. stored in the old AMF cannot be retrieved by the target AMF).

b.	If there is connectivity between the target AMF and the old AMF, the target AMF can fetch any additional information about established PDU sessions etc. stored in the old AMF.

7.	Because the Target AMF has received the keyAmfHDerivationInd indicator, the Target AMF is mandated to run a NAS SMC procedure in step 16 with the UE, to take the new Kamf-1 key into use with the UE.

8.	In step 17, it has been clarified that the Target AMF is mandated to initiate a new primary authentication with the UE to derive a new Kamf-2 when it has received the RR message from the RAN. The new primary authentication procedure is protected by the Kamf-1 key.

9.	In step 18 a and b, the target AMF is mandated to run a new NAS SMC procedure with the UE to take the new Kamf-2 into use with the UE. The target AMF shall include the request to the UE to include the complete Registration Request message in the NAS Security Mode Complete message by setting the flag "request initial NAS flag" in the NAS Security Mode Command message.  The UE includes the complete Registration Request message (sent in step 1) in the NAS Security Mode Complete message to the target AMF. This means that the target AMF can take the Registration Request message received in NAS Security Mode Complete message into use and drop the Registration Request message rerouted via RAN.

10.	Evaluation has been added.

# 4	Detailed proposal

This contribution proposes an update to solution #2 for AMF reallocation procedure when 5G NAS security context is rerouted via RAN. It is proposed to update solution #2 in the study in [1].

**** 1st CHANGE ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[2]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[XX]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) "

**** 2nd CHANGE ****

## 6.2	Solution #2: Security of AMF re-allocation when 5G NAS security context is rerouted via RAN

### 6.2.1	Introduction

This solution address Key Issue #1: "Security of AMF re-allocation procedures ".

In this solution the 5G NAS security context is re-routed via RAN together with the Registration Request (RR) message.

### 6.2.2	Solution details

Before the Initial AMF re-routes the Registration Request (RR) message and the 5G NAS security context, the Initial AMF performs horizontal Kamf derivation of the current Kamf-0 and generates a new Kamf-1 key which is then routed via RAN to the Target AMF together with an indication of horizontal KAMF derivation (i.e., keyAmfHDerivationInd). The current Kamf-0 is not rerouted via RAN to the Target AMF. This would ensure that the Target AMF has no access to the Kamf-0 key used in the Initial AMF/old AMF.

The Initial AMF forwards the RR message, the 5G NAS security context including the Kamf-1 and the keyAmfHDerivationInd indicator unprotected to the Target AMF via RAN.

The new generated Kamf-1 key could be seen as a one-time key for the purpose of the AMF re-allocation. It is worth noting that Kamf-1 is practically useless to a legitimate RAN node since it is a NAS key that has not been put into use by any AMF in the network. The Target AMF would then be mandated to establish a new further key Kamf-2 with the UE, which is not available to the Initial AMF and the RAN.

Because the Target AMF has received the keyAmfHDerivationInd indicator, the Target AMF needs to run a NAS SMC procedure with the UE, to take the new Kamf-1 key into use with the UE. The Target AMF is also mandated to initiate a new primary authentication with the UE to derive a new Kamf-2 when it has received the RR message from the RAN. The new primary authentication procedure is protected by the Kamf-1. This step would ensure that the Initial AMF has no access to the new Kamf-2 key (i.e. the Kamf key used in the Target AMF and the UE).

As the target AMF can determine that a NAS re-route via RAN has taken place, the target AMF needs to restrict the use of the Kamf-1 only for the purpose of sending protected NAS Security Mode Command and Authentication Challenge/Request to the UE, and for receiving protected NAS Security Mode Complete and Authentication Response from the UE.

The Target AMF needs to run a new NAS SMC procedure with the UE to take the new Kamf-2 into use with the UE. The initial AMF needs to indicate to the UE to include the complete Registration Request message in the NAS Security Mode Complete message by setting the flag "request initial NAS flag" in the NAS Security Mode Command message.  The UE includes the complete Registration Request message (sent in step 1) in the NAS Security Mode Complete message to the target AMF. This means that the target AMF can take the Registration Request message received in NAS Security Mode Complete message into use and drop the Registration Request message rerouted via RAN.

Figure 6.2.2-1: AMF re-allocation with NAS message and 5G NAS security context re-route via RAN

Figure 6.2.2-1 shows the solution steps:

Step 1:	 The UE prepares a Registration Request message including a SUCI or a 5G-GUTI and slicing information which could potentially cause an AMF re-allocation. If the UE has a 5G NAS security context (Registration with 5G-GUTI) it includes a protected NAS container in the Registration Request message.

Step 2:	 The RAN forwards the RR message to an Initial AMF.

Step 3a and 3b: These steps may only take place if UE has indicated its 5G-GUTI in the Registration Request message and there is connectivity between the initial AMF and the old AMF (cases 2.a.i and 2.b.i in clause 4.3) . The Initial AMF contacts the old AMF and requests the 5G NAS security context from the old AMF. The old AMF may perform horizontal Kamf derivation of the Kamf key. 

If there is no connectivity between the initial AMF and the old AMF (cases 2.a.ii and 2.b.ii) and the UE has indicated its 5G-GUTI in the Registration Request message, then steps 3a and 3b are skipped and the initial AMF requests the UE identity SUCI from the UE in step 4 and then initiates primary authentication in step 5.

Step 4: The initial AMF may request UE identity SUCI from the UE.

Step 5:	 The Initial AMF may initiate a new primary authentication. This step is optional. This step is needed if the UE has indicated its SUCI in the Registration Request message

Step 6:	 The Initial AMF initiates a NAS SMC. This step takes place if a prior primary authentication has taken place or if the old AMF has performed horizontal Kamf derivation of the Kamf key. The Initial AMF may include the request to the UE to include the complete Registration Request message by setting the flag "request initial NAS flag" if the old AMF has performed horizontal Kamf derivation of the Kamf key or the Registration Request included the UE SUCI.

Step 7:	 The UE includes the complete RR message sent in step 1 in the NAS Security Mode Complete message. The RR message is both integrity protected and encrypted.

Step 8:	 If the Initial AMF needs UE's subscription information to decide whether to reroute the Registration Request and UE's slice selection subscription information was not provided by old AMF, the AMF selects a UDM as described in TS 23.501 [2], clause 6.3.8. the Initial AMF sends Nudm_SDM_Get to UDM.

Step 9:	 The UDM responds to Initial AMF with a Nudm_SDM_GetResponse. The AMF gets the Slice Selection Subscription data including Subscribed S-NSSAIs. The UDM responds with slice selection data to Initial AMF.

Step 10: If there is a need for slice selection, (see clause 5.15.5.2.1 of TS 23.501 [2]), e.g. the Initial AMF cannot serve all the S-NSSAI(s) from the Requested NSSAI permitted by the subscription information, the Initial AMF invokes the Nnssf_NSSelection_Get service operation from the NSSF by including Requested NSSAI.

Step 11: The NSSF performs the steps specified in point (B) in clause 5.15.5.2.1 of TS 23.501 [2]. The NSSF responds to Nnssf_NSSelection_Get to the Initial AMF.

Step 12: The Initial AMF decides to reroute the RR message to a Target AMF via RAN. The Initial AMF optionally performs horizontal Kamf derivation of Kamf-0 to generate a new Kamf-1. This step would ensure that target AMF has no access to the Kamf-0 key used in Initial AMF. If the Initial AMF performs horizontal Kamf derivation then the initial AMF resets the corresponding uplink and downlink NAS COUNTs to 0.

Step 13: The Initial AMF forwards the complete Registration Request message, the 5G NAS security context including the uplink/downlink NAS COUNTs, new Kamf-1 and the keyAmfHDerivationInd indicator to the RAN.

Step 14: The RAN forwards the complete Registration Request message, the 5G NAS security context and keyAmfHDerivationInd indicator to the target AMF.

Step 15. 	After receiving the Registration Request,

If SUCI is included in the Registration Request, the target AMF skips step 15 (as no additional information about established PDU sessions etc. is stored in the old AMF).

If a 5G-GUTI is included in the Registration Request and the target AMF has received a 5G NAS security context and potentially a keyAmfHDerivationInd indicator, then:

-	If there is no connectivity between the target AMF and old AMF (cases 2.a.ii and 2.b.ii) , the target AMF skips step 15 (and any additional information about established PDU sessions etc. stored in the old AMF cannot be retrieved by the target AMF).

-	If there is connectivity between the target AMF and the old AMF(cases 2.a.i and 2.b.i), the target AMF can fetch any additional information about established PDU sessions etc. stored in the old AMF.

Editor's Note: How the solution works if the old AMF provides current 5G Security context to the target AMF is FFS.

Step 16: If the target AMF has received the keyAmfHDerivationInd indicator, then the target AMF needs to run a NAS SMC procedure with the UE, to take the new Kamf-1 key into use with the UE.

Step 17: The target AMF needs to initiate a new primary authentication with the UE to generate a new Kamf-2. The new primary authentication procedure is protected by the Kamf-1. This step would ensure that the Initial AMF has no access to the new Kamf-2 key generated between target AMF and the UE.

Target AMF determines that a NAS re-route via RAN has taken place and the target AMF shall use the Kamf-1 only for the purpose of sending protected NAS Security Mode Command and Authentication Challenge/Request to the UE, and for receiving protected NAS Security Mode Complete and Authentication Response from the UE.

Step 18a-b: The target AMF needs to run a new NAS SMC procedure with the UE to take the new Kamf-2 into use with the UE. The target AMF needs to include the request to the UE to include the complete Registration Request message in the NAS Security Mode Complete message by setting the flag "request initial NAS flag" in the NAS Security Mode Command message.  The UE includes the complete Registration Request message (sent in step 1) in the NAS Security Mode Complete message to the target AMF. This means that the target AMF can take the Registration Request message received in NAS Security Mode Complete message into use and drop the Registration Request message rerouted via RAN.

### 6.2.3	Evaluation

In this solution there is one more additional NAS SMC in the target AMF after the Registration Request message and the 5G NAS security context including the new Kamf-1 key has been re-routed via RAN, in order to take the new Kamf-1 key into use by the UE and target AMF (after the new horizontal Kamf derivation of Kamf-0 to generate a new Kamf-1 in Initial AMF).

Horizontal Kamf derivation in the initial AMF provides backward security so the target AMF has no access to the Kamf-0 and its corresponding NAS key used between the UE and the initial AMF. The initial AMF has access of the new Kamf-1 key re-routed via RAN to the target AMF, but after the target AMF has taken the new Kamf-1 key into use with the UE by running a NAS SMC, the target AMF can initiate a new protected Authentication procedure with the UE in order to generate a new Kamf-2 key shared with the UE, which the initial AMF has no access to. By running a new NAS SMC procedure between target AMF and UE to take the new Kamf-2 key into use, forward security is provided.

This solution has the following impact:

UE:

Editor's Note: UE impact is FFS

AMF:

This initial AMF needs to perform horizontal Kamf derivation of the Kamf before forwarding the 5G NAS security context together with the complete Registration Request message on the N2 interface to the RAN. The target AMF needs to process the new Reroute NAS message. The target AMF also needs to perform an authentication request in order to produce its own security context.

RAN:

-	This solution has impact on RAN and N2 interface. The REROUTE NAS REQUEST message is defined in TS 38.413 [XX] and the Initial AMF includes the INITIAL UE MESSAGE into the REROUTE NAS REQUEST message to RAN, but the REROUTE NAS REQUEST message needs to be updated to include the 5G NAS security context as well as it is currently not supported. Also the RAN needs to forward the 5G NAS security context to the Target AMF together with the INITIAL UE MESSAGE.

This solution proposes to route the 5G NAS security context via RAN and exposes the Kamf-1 key to the RAN which may impose a security threat so that an attacker in RAN can derive further NAS security keys and initiate attacks on the core network and the UE. Such exposure of NAS keys to RAN has not been done before and has risks. The risks were minimized by the following design:

A.	The RAN is still a trusted network entity according to TS 33.501[2], clause 5.3.8.

B.	The Kamf-0 key is never exposed to RAN. Due to performing horizontal Kamf derivation of the Kamf-0 key in the initial AMF and only providing the new horizontally derived Kamf-1 to the RAN, the new generated Kamf-1 key could be seen as a one-time key for the purpose of the AMF re-allocation. It is worth noting that Kamf-1 is practically useless to a legitimate RAN node since it is a NAS key that has not been put into use by any AMF in the network.

C.	The target AMF needs to take the new generated Kamf-1 key received from RAN into use with the UE by running a NAS SMC procedure prior to initiating a primary authentication immediately thereafter in order to replace the Kamf-1 key received from the RAN with a new Kamf-2 key. The Kamf-1 key is invalidated as soon as the Kamf-2 key is activated after a new primary authentication. So, the Kamf-1 key becomes immediately useless.

D.	Before the new primary authentication procedure is initiated in order to replace the Kamf-1 key with a new Kamf-2 key, there are risks as an attacker in RAN which has access to Kamf-1 key could misuse the key and perform attacks on the UE and the target AMF. As the target AMF can determine that a NAS re-route via RAN has taken place, the target AMF can restrictively use the Kamf-1 only for the purpose of sending protected NAS Security Mode Command and protected Authentication Challenge/Request to the UE, and for receiving protected NAS Security Mode Complete and protected Authentication Response from the UE.

**** END OF CHANGES ****