3GPP TSG-SA3 Meeting #104-e-Ad-hoc 	S3-213457r01

- e-meeting, 27 - 30 September 2021												     Revision of S3-XXXXXX

- Source:	Alibaba Group

- Title:	Conclusions for KI#2

- Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

This contribution proposes conclusions for KI#2 in TR 33.857.

# 2	References

[1]	3GPP TR 33.857: “Study on enhanced security support for Non-Public Networks;(NPN) (Release 17)”

# 3	Rationale

This contribution proposes the conclusion for KI#2: Provisioning of Credentials.

Solution #16 proposes a mechanism in which the UE is provisioned with SNPN credentials from PS after the UE registers with the O-SNPN with restricted access and completes authentication with DCS via User Plane.

This contribution suggests to take the approach for the normative work to support the UE with non-3GPP credentials to complete UE onboarding.

# 4	Detailed proposal

***	BEGIN OF CHANGE	***

## 7.2	Conclusions on KI #2: Provisioning of Credentials

The following principles are concluded for normative work:

- In case that the PS can interact with the DCS, the PS can provision the UE and the UDM of SNPN with the SNPN credential upon the receipt of authentication result from DCS which authenticates the UE for provisioning. The PS can obtain encryption and integrity keys derived from default UE credential for the distribution of SNPN credential to the UE.

***	END OF CHANGE	***