3GPP TSG-SA3 Meeting #105-e 	S3-21xxxx

- e-meeting, 8 - 19 November 2021														Revision of S3-214174

- Source:	Xiaomi

- Title:	ProSe: Conclusion for Key Issue #17

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed conclusion for key issue #17 of TR 33.847.

# 2	References

[1]	3GPP TR 33.847 v0.8.0		Study on security aspects of enhancement for proximity based services in the 5G System (5GS)

# 3	Rationale

Solution #40 addresses all the potential requirements in key issue #17. This pCR proposes to make a conclusion for key issue #17 in TR 33.847 [1] based on solution #40.

# 4	Detailed proposal

*************** Start of the Change ****************

## 7.17	Key Issue #17: Supporting security policy handling for PC5 connection of 5G ProSe services

For PC5 security policy handling between UEs for ProSe services, solution #40 is adopted as the baseline for normative work, based on which the following conclusions are drawn:

-	For ProSe direct services, the PC5 security policy handling defined in TS 33.536 [8] can be reused.

-	For ProSe relay services applying CP-based U2N relay security solutions, the security policies are provisioned by the PCF.

NOTE:	Provisioning of PC5 security policies from the ProSe application server to the UE is performed on the application layer hence out of 3GPP scope.

-	For ProSe relay services applying UP-based U2N relay security solutions, the security policies are provisioned by the 5G PKMF.

NOTE:	5G PKMF may get the security policies in different ways (e.g., pre-configured or obtained from other NFs) and this will be decided in the normative phase

-	PC5 CP security policies for ProSe relay services shall aways be configured as “REQUIRED”.

-	PC5 UP security policies for ProSe relay services shall be configured based on the security requirements of the relay services.

*************** End of the Change ****************