- 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222485

- e-meeting, 10th – 14th October, 2022

- Source:	Huawei, HiSilicon

- Title:	New solution to KI#3

- Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in 33.887.

# 2	References

NA

# 3	Rationale

This contribution introduces a procedure to authenticate the AUN3 device by resuing the 7A.2.4 of TS 33.501. The different part comparing to 7A.2.4 is replacing the N5CW with AUN3, replacing the trusted non-3GPP access with 5G-RG, and replacing the TWIF with W-AGF.  By this way, the 5GC will not be impacted.

# 4	Detailed proposal

********* Begin 1st change*********

## 6.Y	Solution #Y: authentication for AUN3 devices

### 6.Y.1	Introduction

This solution addresses KI#1 by reusing existing mechanism.

### 6.Y.2	Solution details

Figure 6.Y.2 AUN3 authentication procedure

0.	The 5G-RG has established the security with W-AGF.

1. The AUN3 device connects to the 5G-RG

2. The 5G-RG sends an EAP-Req/Ideitity message to the AUN3 device.

3. The AUN3 device calculates the SUCI and sends EAP-Res/Identity message with SUCI to the 5G-RG, and the 5G-RG forwards the EAP-Res/Identity message to the W-AGF using W-CP protocol.

4-11. If the 5GC belongs to a PLMN only EAP-AKA’ authentication method can be used. If the 5GC belongs to a SNPN, different types of EAP authentication methods are allowed.

Editor’s Note: the step 8 in the figure needs to be updated.

12. the W-AGF generates the PMK from the KWAGF, and sends the PMK to the 5G-RG for establishing the security over the air.

### 6.Y.3	Evaluation

No impact on 5GC.

******** End of change*********