- 3GPP TSG-SA3 Meeting #108Adhoc-e	S3-222487

- e-meeting, 10th – 14th October, 2022

- Source:	Huawei, HiSilicon

- Title:	E2E solution in L2 Relay

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution is accepted by the group.

# 2	References

NA

# 3	Rationale

This solution proposes to establish a PC5 link between the Source UE and the Target UE.

# 4	Detailed proposal

********* Begin 1st change*********

## 6.Y	Solution #Y: End-to-end security in L2 Relay

### 6.Y.1	Introduction

This solution addressed the requirement “The 3GPP system shall support a means to establish a secure connection between the source UE and the target UE in the UE-to-UE relay scenario” in KI#2 by in L2 relay case. The solutions proposes to usePer-hop links setup first, and then the source UE and the target UE setup another PC5 link, i.e. the PC5-S and PC5-U can be used directly between the source UE and the target UE.

### 6.Y.2	Solution details

Figure 6.Y.2-1: Layer2 relay End-to-End security setup procedure

1.	The source UE discovers a Relay UE using Model A or Model B discovery as concluded in TR 23.700-33[2].

2.	The Source UE and the Relay UE performs PC5 link setup procedure.

NOTE 1:	The detail of this procedure is not in the scope of this solution.

3. The Relay UE discovers a target UE using Model A or Model B discovery specified in TS 33.503[6].

4.	The Relay UE and the target UE performs PC5 link setup procedure.

NOTE 2:	The detail of this procedure is not in the scope of this solution.

5.	The Source UE and the Target UE setup a PC5 link via L2 relay as described in TS 33.503[6]. By this way,  End-to-end security is established between the Source UE and the Target UE.

Editor’s Note: whether perform both hop-by-hop link security and E2E security in U2U relay is FFS

Editor’s Note: details of e2e security establishment are FFS.

### 6.Y.3	Evaluation

TBD

******** End of change*********