3GPP TSG-SA3 Meeting #108e-adhoc	S3-222577

- e-meeting, 10th – 14th October 2022

- Source:	vivo

- Title:	New solution for authentication and authorization of PINE

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR to TR 33.882.

# 2	References

[1]	3GPP TR 33.882: "Study on personal IoT networks security aspects ".

# 3	Rationale

This solution describes a way for network triggering the authentication and authorization for a PINE based on EAP framework.

# 4	Detailed proposal

****Start of Change (new text)****

- 6.X	Solution #X: Authentication and authorization for PINE

- 6.X.1	Introduction

This solution addresses the KI#1 "Authentication and authorization for PINE".

- 6.X.2	Solution details

The procedure describes how 5GC requests the authentication and authorization for a PINE.

Figure 6.X.2-1: Authentication and authorization for PINE

1.	The PINE initially connects to the PEGC, the PEGC indicates to the PEMC of the connection. The PEMC authorizes the PINE connecting to the PEGC based on local configuration, e.g., whether a device with the MAC address is allowed to connect to the PEGC. If success, the PEMC indicates the PEGC that the PINE needs authentication and authorization based on local configuration, e.g., whether the device is a sensitive property and is able to be authenticated.

2.	[Optional] The PINE may disconnect the PEGC and sometime reconnect the PEGC again. In this case, if the PEGC still holds the configuration of the PINE, the PEGC may not indicates to the PEMC for authorization, this can avoid to frequently disturb the PEMC, or the PINE can still access the PIN when PEMC is not reachable.

The PEGC may report the IP address of the PINE to the SMF.

3.	If the PINE needs authentication and authorization, the PEGC sends PDU Session Modification Request to the SMF corresponding to the PDU Session related to the PIN.

Editor's note:	How the PEGC deteremines a PINE needs AA is FFS.

4.	The SMF may request identity from PINE using EAP messages over NAS same as described in steps 9-10 of clause 11.1.2 in TS 23.501 [X], the PEGC forwards the EAP messages between the SMF and the PINE.

5.	The SMF sends the EAP message to the external DN-AAA, then the external DN-AAA and the PINE exchange EAP messages via the SMF, UPF, and the PEGC same as described in steps 11-13 of clause 11.1.2 in TS 33.501 [X] with following difference:

-	The PEGC exchanges the EAP messages with the PINE.

6.	If the authentication succeeds, the same as described in steps 14 of clause 11.1.2 in TS 33.501 [X] is performed.

7.	The SMF authorizes the PINE based on the information received from external AAA.

8.	The SMF sends PDU Session Modification Ack to the PEGC.

9.	The PEGC authorize the PINE connecting to the PEGC.

Editor's note:	Whether AA context needs to be stored and where to store is FFS.

- 6.X.3	Evaluation

TBD.

****Second of Change****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-88: "Study on Personal IoT Networks"

[X]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

****End of Change****