3GPP TSG-WG SA3 Meeting #109 AH-e	S3-230047

January 16th – 20th, 2023, Emeeting

- Source:	vivo

- Title:	Interim conclusions on KI#1

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR to TR 33.882.

# 2	References

[1]	3GPP TR 33.882: "Study on personal IoT networks security aspects".

# 3	Rationale

This paper proposes interim conclusion for KI#1.

# 4	Detailed proposal

****Start of Change ****

- 7	Conclusions

- 7.X	Conclusions on Key Issue #1

Editor's Note: Further agreed principles are needed.

The following interim principles are agreed for Key Issue #1 "Authentication and authorization for PINE":

1)	The 5GC supports PINE authentication by external DN-AAA over 5G CP (sol#1, #2, #3, #5), or over 5G UP (sol#4), or both.

2)	The SMF is able to authorize which external DN-AAA is allowed or disallowed to perform PINE authentication, which may be per PIN. The configuration of allowed/disallowed external DN-AAA is out of scope. (sol#2)

3)	For 5GC supports PINE authentication over 5G CP, the SMF is able to be an authenticator for authenticating PINE during PDU Session Establishment procedure and PDU Session Modification procedure (sol#1, #2, #3, #5).

4)	The PEGC may support stopping traffic relay for a PINE and redirecting PIN message from the PINE to a PEMC after the PINE connecting to the PEGC without being authenticated. (sol#X)

5)	The PEGC support determining whether the conditional authorization result received from the PEMC is applicable based on the result of the PINE authentication. (sol#X)

****End of Change****