3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230063-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Lenovo

- Title:	Evaluation to Solution#8

- Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

This contribution provides the evaluation for solution #8 in TR 33.887

# 2	References

[1]	3GPP TR 33.887, ‘Study on Security aspects for 5WWC Phase 2’, (Release 18).

# 3	Rationale

The evaluation to Solution #8 has been provided.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.887

*****Start of Change 1*****

### 6.8.3	Evaluation

The solution addresses KI#4 and it enables the following aspects:

UE and TNGF performs mutual authentication during UE TNAP mobility as part of re-authentication by exchanging nonces and verifying a related MAC.

Further impacts related to the solution includes:

For UE TNAP mobility scenario, the UE and TNGF derives new TNAP key using the previously established security context (from the initial primary authentication) i.e., TNGF key and nonce(s) as a freshness parameter. The UE and TNGF need to derive a Re-auth ID, the details of the inputs is upto the normative work.

UE need to construct NAI using Re-auth ID and TNGF information. The TNGF need to find a stored UE context based on Reauth-ID.

TNGF: Need to provide Nonce, TNGF ID to UE during initial registration procedure. Alternatively, an existing TNGF address can be used TNGF ID.

Editor’s Note: Further evaluation if any is FFS.

*****End of Change 1*****