3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230103-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	ZTE

- Title:	Update the KI#6

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the key issue#6.

# 2	References

[1] 	TR 33.740  v0.4.1

# 3	Rationale

It is proposed to update the key issue #6. Some references should be updated. And according to SA2’s conclusion, both layer 2 and layer 3 UE-to-network relay for emergency service should be considered.

# 4	Detailed proposal

********************Start of 1st Change******************

## 5.6	Key Issue #6: Support for Emergency service over UE-to-Network Relaying

### 5.6.1	Key issue details

In TR 23.700-33 [1], key issue #7 is about support of Emergency for UE-to-Network Relaying. According to TS 22.101 [xx], emergency service is defined as citizen to authority services, and it is left to the national authorities to decide whether the network accepts emergency calls e.g. for valid UE only, or for UEs without the SIM/USIM/ISIM.

In the 5G ProSe UE-to-Network relaying, for both layer 2 UE-to-network relay and layer 3 UE-to-nework relay, if there is an emergency request from the remote UE, it implies that the Relay UE needs to be responsible for remote UE's emergency service.

A Remote UE which has no USIM inserted may need to initiate an emergency service with a network over PC5 interface via a Layer 3 UE-to-network relay. This Remote UE can only discover UE-to-network relays which indicates a RSC for emergency in clear text. Also, this Remote UE is not able to establish a secure PC5 link with a UE-to-network relay.

Based on the operator policy, the Relay UE needs to support to establish PC5 communication for emergency service with or without PC5 security.

### 5.6.2	Security threats

Emergency service can’t be fulfilled if a Remote UE cannot be properly identified and establish the PC5 security with the relay UE (if required).

### 5.6.3	Potential security requirements

The 5G system shall support the establishment of PC5 communication for emergency service over UE-to-network relay with or without PC5 security.

********************End of 1st Change******************

*******************Start of 2nd Change******************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-33: "Study on System enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

[3]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[5]	3GPP TS 22.115: " Service aspects; Charging and billing".

[6]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[7]	3GPP TR 33.870: "Study of privacy of identifiers over radio access".

[8]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[9]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[10]	IRTF “CPace, a balanced composable PAKE” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-cpace/

[11]	IRTF “The OPAQUE Asymmetric PAKE Protocol” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-opaque/

[12]	IRTF “SPAKE2, a PAKE” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-spake2/

[13]	IRTF “SPAKE2+, an Augmented PAKE” available online at https://datatracker.ietf.org/doc/draft-bar-cfrg-spake2plus/

[14]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[15]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[xx]	3GPP TS 22.101: "Service aspects; Service principles".

*******************End of 2nd Change******************