3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230160-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	Huawei, HiSilicon

- Title:	Evaluation to solution 15

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes to approve the evaluation to solution 15.

# 2	References

NA

# 3	Rationale

It is proposed to add evaluation for solution #15.

# 4	Detailed proposal

It is proposed to approve the following evaluation to solution 15.

*************** Start of 1st Change ****************

### 6.15.3	Solution evaluation

This solution proposes the 5GC to provide the UE supported authentication methods of UE SN and/or HN during the the registration procedure. And the ECS Authentication mechanism capability is preconfigured as part of ECS Address Configuration Information. Then the EEC can determine the candidate authentication mechanism(s) which is supported by UE, EEC and SN/HN and send the candidate mechanism(s) to the ECS, and the ECS determines the used authentication based on received candidate authentication mechanism(s). The solution also extends the service provioning procedure to send the replayed candicate authentication mechanism for preventing binding down attack because the negotication procedure is not protected.

*************** End of Change****************