3GPP TSG-SA3 Meeting #109AdHoc-e	draft_S3-230290-r3

- Electronic meeting, 16 - 20 January 2023										revision of S3-22xxxx

- Source:	Qualcomm Incorporated

- Title:	Conclusion for key issue #1

- Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution provides a conclusion for key issue #1.

# 2	References

[1]	3GPP TR 33.890 v0.4.0 (S3-224179)

# 3	Rationale

This contribution provides a conclusion for key issue #1.

# 4	Detailed proposal

It is proposed that the following pCR is included in the TR [1].

**** START OF CHANGES ****

## 7.X		Conclusions for Key Issue #1

The following conclusions are agreed for KI#1:

1)	The 3rd party ID may be included in the SIP INVITE received by the originating IMS;

2)	The Originating IMS authorizes the originating UE to use 3rd party IDs. The IMPU and optionally the 3rd party ID is provided to the third party AS which based on the provided IMPU/3rd party ID returns a name card to the IMS subsystem for signing. In case the 3rd party ID is not included in the SIP INVITE, the 3rd party AS depending on the policy associates one name card based on the IMPU, this name card being a specific to a specific IMPU or a default name card.

Editor's Note: how can IMS differentiate whether UEs need to include a default name card or have a call without name card when the 3rd party ID is not included is ffs.

Editor's Note: it’s ffs whether AS can optionally provide the name card based on 3rd party ID and token allocated by itself.

Editor's Note: it’s ffs what does the AS mean, i.e., application server or authorization server.

3)	The name cards signing and verification are based on SHAKEN Reference Architecture and TS 24.229 [4]. More specifically:

-	The Originating IMS signs the RichData/Name Card using STIR/SHAKEN.

-	The Terminating IMS verifies the RichData/NameCard using STIR/SHAKEN.

**** END OF CHANGES ****