3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230330-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	Samsung

- Title:	Resolving EN in solution#4 and updating evaluation (TR 33.739)

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested to approve this pCR for resolving EN and updating evaluation in solution#4 TR 33.739.

# 2	References

NA

# 3	Rationale

This contribution proposes to resolve the following Editor’s Note:

Editor’s Note: Which core network NF stores the supported authentication method details is FFS.

Supported authentication methods are exposed by the NEF. The ECS obtains information from the NEF. Then, the ECS determines the authentication method based on the security capability of the HN/SN.

Editor's Note:	Further evaluation is FFS.

With the above updates the evaluation text is further refined. Hence, the above Editor’s Note is removed.

# 4	Detailed proposal

*** Start Change ***

## 6.4	Solution #4: Authentication mechanism selection between EEC and EES

### 6.4.1	Solution overview

This solution proposes a mechanism to select one of the authentication methods supported by Home network and indicate the same to the EEC addressing the security requirements of key issue#2.2.

### 6.4.2	Solution details

Figure 6.4.2-1: EES indicates the supported authentication method in EES profile during the EES registration

- The EES sends the EES registration request to the ECS. The request from the EES includes the EES profile, EES security credentials and the supported authentication method(s) by the EES.

- Upon receiving the request from the EES, the ECS verifies the security credentials of the EES and stores the EES registration information obtained in step 1.

- The ECS sends an EES registration response indicating success or failure of the registration operation.

During service provisioning, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, the required information (e.g. URI, IP address) for establishing a connection to the EES and authentication method selected.  ECS selects the authentication method between the EEC and EES based on the authentication method(s) supported by the UE, EEC, EES, HN and SN. ECS gets the information on the home network and serving network supported authentication method(s) from the core network.  The NEF exposes the network supported authentication method.

***Next Change ***

### 6.4.3	Solution evaluation

This solution addresses the security requirement from Key issue#2.2 i.e., selection of authentication mechanism for authentication procedures between EEC and EES.

In this solution the decision or selection of the authentication method is determined based on the UE, EEC, home network and serving network. EES indicates the supported authentication method(s) to the ECS in the EES profile. The NEF exposes the authentication method supported by the network. The solution requires a new service provided by the NEF. The ECS then selects the authentication method based on UE, EEC, EES, HN and SN and then indicates the selected authentication method to the EEC in the service provisioning response.

***End Change ***