3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230342-r6

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Adding conclusion on KI#1

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes conclusion on KI#1 in TR 33.738

# 2	References

[1]	3GPP TR 33.738 “Study on security aspects of enablers for Network Automation for 5G - phase 3”

[2]	3GPP TR 23.700-81 “Study of Enablers for Network Automation for 5G System (5GS); Phase 3”

# 3	Rationale

This pCR proposes to use the ideas of solution #5 of TR 33.738 [1] and conclusions of KI#3 in TR 23.700-81 [2] to conclude KI#1 in TR 33.738 [1].

The evaluation of solution #8 in TR 33.738 [1] indicates that authorization is based on the extension of existing SBA security schema by adding the requested data type for the access token request and subsequently to the access token. Thus, the NRF acts as the authorization server. Although the proposal is technically valid, it is not completely aligned with the architecture conclusions in [2] with respect to data and analytics exchange in roaming case, where it is stated that NWDAF is used as an entry point to exchange analytics in roaming scenario, and it authorizes the requests according to operator policies and user consent and it filters the information exposed in reply to the request. For that, new services to be accessed by the peer PLMN are offered in the NWDAF.

There is not yet evaluation of solution #14, so that solution has not been considered in the proposal for KI#1 conclusion in [1] at the moment.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

***BEGIN OF First CHANGE***

# 7	Conclusions

## 7.X	Conclusion on Key Issue #1 "Protection of data and analytics exchange in roaming case”

For Key Issue #1, it is recommended to use the following principles as the baseline for protection of data and analytics exchange in roaming case:

- The introduction of a new service for authorization in the NWDAF as an entry point to exchange analytics in roaming scenarios between HPLMN and VPLMN. This new service will act as a proxy for the analytics consumers (NWDAF) in other PLMNs.

- The NRF uses token-based authorization to restrict the access to services of NWDAF from outside the own PLMN only to the new service of the NWDAF.

- *** END OF First CHANGE ***