3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230343-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution for authorization in FL

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes a new solution for KI#2 in TR 33.738. It is requested to approve this proposal.

# 2	References

# 3	Rationale

This contribution proposes a solution to address Key Issue #2 of TR 33.738.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.738.

**************START OF CHANGES***********

### 6.X.1	Introduction

Based on existing OAuth 2.0 schema specified for SBA, the solution proposes an authorization mechanism for FL NWDAF server(s) to consume the resources and data of the NWDAFs acting as FL clients, based upon the FL role information and the analytics ID and other data details registered at the NRF during the NF profile registration phase.

### 6.X.2	Solution details

The figure 6.X.2-1 represents the authorization workflow of the solution:

Figure 6.X.2-1 Authorization workflow for FL

Step 1: NWDF FL member registers in the NRF with the following attributes:

a) “FL Process Role”, i.e., whether the NWDAF registers as a FL server or as FL client for a particular Analytics Id

b) “Allowed NF(s) (NF type and NF Instance id) as FL server(s) for a particular Analytics Id, if the NWDAF has registered as FL client in NRF

When new allowed FL server NWDAFs are introduced, the FL member NWDAF needs to update its list of allowed server NWDAFs.

NOTE: Other data details can be added and are subject to the implementation, such as sensitivity of the data used for training (e.g., it may contain privacy data of subscribers), which are to be decided by the operator.

Step 2: FL server sends a discovery request to the NRF to retrieve the information for the NWDAF registered as FL client for a given Analytics Id.

Step 3: FL server sends an access token request for each required NWDAF FL client (in the picture a single request is shown)

Step 4: NRF authorizes the request of the NWDAF FL server based upon the information registered in Step 1.

Step 5a: If the NWDAF FL server is authorized to request the NWDAF FL clients to perform the FL operations, the NRF provides the corresponding access tokens for the selected NWDAF FL client member(s).

Step 5b: else, NRF would reject the access token requests.

Step 6: The NWDAF FL server sends the service requests to the NWDAF FL clients with the respective access tokens received in Step 5a.

Step 7: NWDAF FL clients can start the FL process for a given request in case of successful access token verification.

Editor's Note: If user consent may be required for a client to join a FL group is ffs.

NOTE: Authorization process of the NWDAF FL client(s) by the NWDAF FL server is implicit, since the NWDAF FL server sends the requests only to the discovered NWDAF FL clients.

### 6.X.3	Evaluation

TBD

************ END OF CHANGES ******************