3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230359-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	Xiaomi

- Title:	33.893: Update to Solution #2

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed update to solution #2 in TR 33.893.

# 2	References

[1]	TR 23.800-76	 v1.1.0		Study on Architecture Enhancement to support Ranging based services and sidelink positioning

[2]	TS 23.273 v18.0.0		5G System (5GS) Location Services (LCS)

[3]	TR 33.893 v0.4.0			Study on Security Aspects of Ranging Based Services and Sidelink Positioning

# 3	Rationale

According to SA2 conclusion on Network assisted SL Positioning in clause 8.5 of TR 23.700-86 [1], for a SL positioning service triggered by a 5GC NF, the request should be sent to the AMF by the 5GC NF for triggering location service. Then the AMF will select a SL positioning capable LMF which performs positioning protocol with the UE based on the legacy functionality specified in TS 23.273 [2].

In step #11 of solution #2, the 5GC NF sends the SL positioning request to the AMF, which is aligned with SA2 conclusion. In step #12, the AMF should select a SL positioning capable LMF to perform SL positioning procedure. As step #12~#15 are up to the procedure defined in SA2, they are simplified without reflecting the LMF in the flow chart. Therefore, this pCR proposes to update the relevant steps in solution #2 of TR 33.893 [2] with more details for alignment with SA2 conclusion.

With the LMF being involved, the updated solution actually assumes the reuse the existing 5GC-MT-LR procedure supported by the LMF. Hence, the introduction and evaluation for solution #2 are also updated accordingly.

# 4	Detailed proposal

*************** Start of the Change ****************

## 6.2	Solution #2: Authorization of 5GC NF for Ranging/SL positioning service exposure

### 6.2.1	Introduction

This solution addresses Key Issue #2 on Authorization for Ranging/SL Positioning service. Specifically, it addresses the second requirement in KI#2: “The 5G Ranging/SL positioning system shall be able to support the authorization of a network function for triggering Ranging/Sidelink Positioning services and obtaining the location information”.

According to TR 23.700-86 [2], 5GC NF is enabled to initiate SL positioning service to obtain the accurate location of a UE or obtain the distance between two UEs, for which the 5GC NF needs to be authorized at two levels:

- The first level of authorization is for service access, i.e. the 5GC NF should be authorized to request the SL positioning service. This level of authorization could be achieved by existing OAuth token-based authorization defined in TS 33.501 [8].

- The second level of authorization is for preserving UE privacy. This is because, even if the 5GC NF is authorized to request the SL positioning service, it does not mean that the requested service can always be exposed to the 5GC NF. For example, it is possible that the 5GC NF is allowed to request SL positioning info between UE1 and UE2, but may not be allowed to request SL positioning info between UE3 and UE4. Without further check on the authorization info of the involved UEs, there is still the risk that unauthorized SL positioning information could be exposed to the 5GC NF. Therefore, the 5GC NF needs to be further authorized on whether it is allowed to acquire position information of the involved specific UEs.

This solution introduces a method for authorizing the 5GC NF which initiates SL positioning service targeting at specific UEs.

### 6.2.2	Solution details

The solution proposes that, when the 5GC NF checks with the UDM to discover the serving AMF(s) of the UEs, it also needs to check with the UDM about authorization information of the UEs for acquiring SL positioning information of the UEs, based on e.g. the privacy related parameters in UE’s subscription data stored in the UDM/UDR.

UE1 and UE2 refer to SL Reference UE and Target UE respectively for SL Positioning service.

1.	Service authorization and policy/parameters provisioning procedure is performed between UE1/UE2 and the network respectively.

2.	The 5GC NF interacts with the UDM to check the authorization info of UE1/UE2 via Nudm_SDM_Get service operation.

NOTE:	If UE1 and UE2 are managed by different UDMs, the 5GC NF sends the Nudm_SDM_Get message to the corresponding UDMs respectively.

3.	The UDM checks the authorization info of UE1 and UE2 against their subscription data respectively based on the UE1 ID and UE2 ID.

4.	The UDM returns the authorization results of the UEs to the 5GC NF.

5.	The 5GC NF checks the authorization results of both UEs. Since SL Positioning service concerns location of the UE, it could be possible that the authorization info of the UE is location specific (e.g. the UE allows its location to be exposed in area A but does not allow its location to be exposed in area B).

If none of the UE grants or one of the UEs does not grant authorization for Ranging/SL positioning, the 5GC NF aborts the Ranging/SL positioning service.

If both UEs grant authorization without location restriction, the 5GC NF proceeds to step #11.

Conditionally, if both UEs grant authorization which is restricted in a certain area, the 5GC NF proceeds to step #6.

If the 5GC NF is the AMF of the UEs, the 5GC NF skips the steps #6~#9 and proceeds to step #10.

Figure 6.2.2-1:	Authorization of the 5GC NF for Service Exposure

6.	[Conditional] The 5GC NF sends the Nudm_ParameterProvision_Get Request to the UDM for requesting the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

7.	[Conditional] The UDM sends the Namf_Location_ProvideLocationInfo Request to the AMF.

8.	[Conditional] The AMF of UE1/UE2 responds with the Namf_Location_ProvideLocationInfo Response to the UDM which contains the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

9.	[Conditional] The UDM returns the Nudm_ParameterProvision_Get Response to the 5GC NF which contains the coarse location of UE1/UE2 (e.g. TAI or Cell-ID of UE1/UE2).

10.	[Conditional] Based on the coarse location of UE1/UE2, the 5GC NF checks whether the UE1/UE2 is within the area for requesting SL Positioning service.

11. 	If both UEs grant authorization in their current locations, the 5GC NF sends the SL Positioning service request to the AMF of one of the UEs (e.g. UE1) based on the conclusion in clause 8.5 of TR 23.700-86 [2]. If the 5GC NF is the AMF, this step can be skipped.

12~15.	 The AMF selects a SL Positioning capable LMF (not shown in Figure 6.2.2-1) to perform the SL Positioning procedure between the network and the UE.

Alternatively, after the 5GC NF checks the authorization info of both UEs in step #5, if both UEs grant authorization which is however restricted in a certain area, the 5GC NF sends the Namf_Location_ProvideLocationInfo Request to the AMF directly rather than through the UDM. Then the AMF responds the Namf_Location_ProvideLocationInfo Response to the 5GC NF directly rather than through the UDM.

NOTE:	The area granularity could be detailed in the UE privacy profile for SL positioning in UE subscription.

### 6.2.3	Evaluation

This solution addresses the second requirement “The 5G Ranging/SL positioning system shall be able to support the authorization of a network function for triggering Ranging/Sidelink Positioning services and obtaining the location information” in KI#2.

On top of the existing authorization of the 5GC NF on service level, the authorization in this solution is further performed on specific UE level, which ensures the privacy of all involved UEs in a service, as Ranging/SL Positioning services request UE location information which is privacy sensitive.

This solution relies on the requesting 5GC NF itself to check with the UDM for the authorization information of the involved UEs. Therefore, only the requesting 5GC NF is impacted in this solution, which is anyway impacted by the requirement of triggering the AMF to select a SL Positioning capable LMF for SL Positioning services.

*************** End of the Change ****************