3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230396-r1

- Electronic meeting, 16 - 20 January 2023										    Revision of S3-22xxxx

- Source:	Xiaomi

- Title:	Remove the Editor's Note and evaluate the solution #7 in TR 33.740

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to evaluate the solution #7 in the FS_5G_ProSe_Ph2 study TR 33.740

# 2	References

[1]	3GPP TR 33.740 v0.4.0: "Study on security aspects of Proximity Based Services (ProSe) in 5G System (5GS) phase 2"

[2]	3GPP TS 23.304 v18.0.0: "Proximity based Services (ProSe) in the 5G System (5GS)"

# 3	Rationale

This pCR proposes to address the Editor’s Note and evaluate the solution #7 in TR 33.740 [1].

Editor’s Note: Solution details will be studied based on SA2’s conclusion about the KI#1 in TR 23.700-33 [2].

The procedure for 5G Prose Communication via 5G Prose Layer-3 UE-to-UE Relay is specified in Clause 6.7.1 of the TS 23.304 [2], which is aligned with this solution.

It is proposed to remove this Editor’s Note.

# 4	Detailed proposal

*************** Start of the Change ****************

## 6.7	Solution #7: Non-network-assited Security Establishment Procedure for 5G ProSe Layer-3 UE-to-UE Relay

### 6.7.1	Introduction

The solution addresses Key Issue #2: Security of UE-to-UE Relay. It largely reuses the mechanism of Direct Security Establishment procedure defined in TS 33.503 [6] to ensure the security of UE-to-UE Relay Communication.

For Layer-3 UE-to-UE Relay, the full security of a UE-to-UE PC5 link depends on the security of two separate PC5 links, i.e. the link between the Source UE and UE-to-UE Relay and the link between UE-to-UE Relay and Target UE. The security of these two separate PC5 link relies on the security materials (i.e. the long term credential), which can be pre-configured on the 5G ProSe UE (incl. Source UE, Target UE and UE-to-UE Relay) or provisioned by the network during the service authorization procedure. In other words, all the ProSe UEs can obtain the security materials without the assistance of network. Therefore, the Source UE and the Target UE can establish the UE-to-UE PC5 link via Layer-3 UE-to-UE Relay regardless of whether they are within or out of network coverage.

### 6.7.2	Solution details

Figure 6.7.2-1: Non-network-assited PC5 link security establishment procedure for 5G ProSe Layer-3 UE-to-UE Relay

0.	The long term credential and long term credential ID are associated with RSC, which could be pre-configured on the 5G ProSe UE (incl. Source UE, Target UE and UE-to-UE Relay) or provisioned by the network e.g. during Service Authorization and Provisioning procedure before the U2U discovery procedure.

Note 1:	How to pre-configure the long term credential to UE is out of the 3GPP scope.

1.	The Discovery & Relay Selection procedure is performed between the peer UEs and the UE-to-UE Relay.

Note 2:	It is assumed that after the Discovery & Relay Selection procedure, the Source UE (UE1) and the Target UE (UE2) can discover each other by selecting the same UE-to-UE Relay.

2.	The Source UE sends a Direct Communication Request that contains the long term credential ID, Source UE’s security capabilities, RSC and nonce 1 to the UE-to-UE Relay as specified in the TS 33.503 [6]. The message may also include a Knrp ID if the Source UE has an existing Knrp with this UE-to-UE Relay for the same RSC.

3.	The UE-to-UE Relay may initiate a Direct Auth and Key Establish procedure with Source UE to generate the Knrp. If the Knrp ID is included in the Direct Communication Request, this step is skipped.

4.	The UE-to-UE Relay derives the session key (KNRP-SESS) from KNRP and then derives the confidentiality key (NRPEK) (if applicable) and integrity key (NRPIK) based on the PC5 security policies as specified in TS 33.503 [6]. The UE-to-UE Relay sends a Direct Security Mode Command message to the Source UE. This message also includes the chosen security algorithm and nonce 2.

5.	The Source UE responds with a Direct Security Mode Complete message to the UE-to-UE Relay.

6.	The UE-to-UE Relay sends a Direct Communication Request that contains the long term credential ID, the chosen security algorithm, RSC and nonce 1' to the Target UE as specified in the TS 33.503 [6]. The message may also include a Knrp ID' if the UE-to-UE Relay has an existing Knrp' with this Target UE under the same RSC.

Note 3:	How the U2U relay determiness to send DCR to UE2 is determined by SA2.

7.	The Target UE may initiate a Direct Auth and Key Establish procedure with UE-to-UE Relay to generate the Knrp'. If the Knrp ID' is included in the Direct Communication Request, this step is skipped.

8.	The Target UE derives the session key (KNRP-SESS’) from KNRP’ and then derives the confidentiality key (NRPEK') (if applicable) and integrity key (NRPIK') based on the PC5 security policies. The Target UE sends a Direct Security Mode Command message to the UE-to-UE Relay. This message also includes the nonce 2'.

9.	The UE-to-UE Relay responds with a Direct Security Mode Complete message to the Target UE.

10. The Target UE sends the Direct Communication Accept message to the UE-to-UE Relay.

11.	Only receiving the Direct Communication Accept message from the Target UE, the UE-to-UE Relay then responds with the Direct Communication Accept message to the Source UE.

12.	The secure Layer-3 PC5 link between the Source UE and the Target UE via the UE-to-UE Relay is established. The UE-to-UE Relay can forward the traffic between the peer Prose UEs.

### 6.7.3	Evaluation

This solution addresses KI#2, which is based on the mechanism of Direct Security Establishment procedure defined in TS 33.503 [6].

This solution is aligned with the SA2’s procedure defined in TS 23.304 [8].

This solution can ensure that the Source UE and the Target UE can establish the UE-to-UE PC5 link via Layer-3 UE-to-UE Relay regardless of whether they are within or out of network coverage.

In this solution, the long term credential is used as the root key for UE-to-UE Relay link establishment. The long term credential can be pre-configured on the 5G ProSe UE (incl. Source UE, Target UE and UE-to-UE Relay).

This solution fulfills all the security requirements of KI#2. The solution can protect security (i.e., the integrity, confidentiality, and replay protection) of user-plane and control-plane messages.

Editor’s Note : further evaluation is FFS.

*************** End of the Change ****************