3GPP TSG-SA3 Meeting #122	S3-252380

- Fukuoka, Japan, 19 – 23 May 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR Solution for KI#3: Paging with AIOT group ID

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_sec

Comments

So far, defense mechanisms on AIoT device identifiers privacy have only considered the identity transmitted in the inventory response. To protect against binary enumeration of the device ID by paging, a different approach is required. This solution decouples the group ID from the device identifier, enabling arbitrary semantics of the bits of the group ID in order to allow efficient paging, while still protecting privacy of the device.

* * * First Change * * * *

## 6.XX	Solution #XX: temporary ID based AIoT device privacy protection

### 6.XX.1	Introduction

This solution addresses the security requirement of KI#3: Privacy by protecting the AIoT permanent device identifier. This solution introduces an AIoT device group ID to provide the network with a mechanism to use filtering information without disclosing any part of the permanent AIoT device identifier over the air interface. The AIoT device group ID is allocated by the AIOTF.

### 6.XX.2	Solution details

This solution describes the allocation and usage of the AIoT device group identifier.

There are two phases of operation to this solution, setting the temporary group ID, and using it in paging.

Phase A: Setting of group ID (cf. Fig. 6.XX.2-1):

0. The device may be pre-provisioned with an AIoT device group identifier based on customer definition.

1. The AF requests the AIOTF to assign a new group ID to a device (or group of devices)

1a. The AIOTF verifies the AF is authorized to update the group ID.

2. The AIOTF sends 	an AIOT command service request (assignGroup) to the device, requesting it to write a new device group ID.

3. The AIOT device responds with an acknowledgement of successful update of group ID. The AIOTF updates the AIoT device profile via ADM.

4. The AIOTF sends an acknowledgement to the AF.

A device may support a small number of multiple concurrently active group IDs. In that case the AIOTF needs to indicate in the service request which group ID to replace.

Figure 6.XX.2-1: System architecture and security assumption

Phase B: Using group ID for paging

1. Instead of using permanent AIoT device ID identifier with mask for paging the device, the AIOTF uses the group ID with a group ID mask. The reader may include a group ID matching bit, so that the AIoT device knows it should consider group IDs instead of the permanent AIoT device identifier. The AIoT device shall not respond to paging for permanent AIoT device identifier with a mask.

2. If any of the group IDs stored on the device matches or partially matches based on the the group ID mask used in the paging, the device responds to the paging.

How the device is ID is concealed in the response to the paging request is an orthogonal problem, which is solved by another solution.

### 6.XX.3	Evaluation

This solution  addresses security requirement in KI#3 for paging.

This solution requires the AIoT device to store the group ID allocated by the Network

Note:	 The evaluation is not complete.

* * * End of Changes * * * *