3GPP TSG-SA3 Meeting #122																			    S3-252430

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0109 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Nested API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung, Xiaomi, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is proposed to specify authorization in nested API invocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause in TS 33.122 to be added to introduce authorization aspects for nested API invocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be an authorization mechanism specific to nested API invocation case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implements the living document S3-252407 approved in SA3#122. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * Start of the Changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[5]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[6]	IETF RFC 7519: "JSON Web Token (JWT)".

[7]	IETF RFC 7515: "JSON Web Signature (JWS)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	Void

[10]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[11]	IETF RFC 7636: " Proof Key for Code Exchange by OAuth Public Clients".

[xx]	IETF RFC 8693: "OAuth 2.0 Token Exchange".

* * * Next Change * * * *

## 6.X	Authorization procedure in a nested API invocation

The nested API invocation scenario is a scenario where an API invocation towards a first API exposing function (AEF-1) triggers that this API exposing function (AEF-1) requests an API invocation towards a second API exposing function (AEF-2), i.e., AEF-1 becomes API Invoker towards AEF-2. Both belong to the same API provider domain. In this scenario, the CAPIF may reduce the authorization information inquiries for a nested API invocation using procedure described in this clause.

The authorization of API invocation triggered towards the second API exposing function uses token exchange procedure as specified in IETF RFC 8693 [xx], where the access token of the API invoker to be used towards AEF-1 is used as the subject token as per the IETF RFC 8693 [xx]. AEF-1 before triggering API invocation towards AEF-2, invokes the token exchange request towards the CCF by sending the subject token to receive a new access token. AEF-1 uses the received new access token towards AEF-2 for nested API invocation.

Figure 6.X-1: Authorization for nested API invocation

1.	API invoker - 1 invokes the AEF-1 service by using the access token obtained from the CCF/Authorization Function.

2.	Based on the service API invocation request, AEF-1 verifies the access token and decides to invoke another service API exposed by AEF-2.

3a.	AEF-1 sends a token exchange request message to CCF, to get a token to invoke the service API in AEF-2. The token exchange request message contains grant_type, the access token as the subject_token, optionally an actor_token, and optionally scope for the desired scope of the requested token. The grant-type is set as the token-exchange.

3b.	The CCF validates whether the requesting AEF-1 is allowed for accessing the requested service API of AEF-2. Also, the CCF validates the access token in the request message that is provided by the API invoker to AEF-1.  After successful validation, the CCF generates a new access token to allow AEF-1 to invoke the service API on AEF-2. The CCF checks whether the received token is an RNAA token and if so issues the new token as an RNAA token which includes the AEF ID in the (act) actor claim optionally, the API invoker ID, the resource owner ID and the scope, in addition to the standard access token claims as specified in clause C.2.2. The CCF responds to AEF-1 with the token exchange response message including the newly issued access token and optionally the scope (if required) for service API invocation on AEF-2.

4.	AEF-1, sends a service API invocation request to AEF-2 with the authorization information i.e., access token received in step 3b.

5.	AEF-2 checks whether the API invoker is authorized to invoke the requested service API based on the received token and if allowed, AEF-2 sends a service API invocation response.

6.	The API invoker receives from AEF-1 the service API invocation response after the service API invocation from AEF-2.

* * * End of the Changes * * * *