# 3GPP TSG-SA3 Meeting #122																			S3-251860

# Fukuoka, Japan, 19 – 23 May 2025

Source:	NIST

Title:	Cryptographic Inventory Table update for MIKEY-SAKKE

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This document provides entries for tables 4.2-1 and 4.3-1 corresponding to clause 4.4.9 MIKEY-SAKKE.

MIKEY-SAKKE is a key exchange protocol designed to establish encryption keys for real-time multimedia communications without requiring traditional public key infrastructure. MIKEY-SAKKE is specified in IETF RFC 6509

https://datatracker.ietf.org/doc/rfc6509/

SAKKE is the key encapsulation mechanism used by MIKEY-SAKKE, specifically it is used to securely transmit a shared secret from the sender to the receiver using identity-based encryption (IBE). The SAKKE is specified in IETF RFC 6508

https://datatracker.ietf.org/doc/html/rfc6508

MIKEY-SAKKE uses ECCSI, for elliptic curve-based certificateless signatures, primarily intended for use with Identity-Based Encryption (IBE) schemes. The ECCSI cryptographic algorithm is based upon ECDSA. The only difference between ECDSA and ECCSI is that in ECCSI the 'public key', Y, is derived from the Signer ID by the Verifier (whereas in ECDSA the public key is fixed). It is therefore assumed that the security of ECCSI depends entirely on the secrecy of the secret keys. ECCSI is specified in IETF RFC 6507.

https://datatracker.ietf.org/doc/rfc6507/

* * * First Change * * * *

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) | IETF RFC 6509 Appendix A | SHA256 | Hash |

* * * Next Change * * * *

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) | IETF RFC 6507 | ECCSI | Digital signature |
|  | IETF RFC 6508 | SAKKE | Key agreement |

* * * End of Changes * * * *