3GPP TSG-SA3 Meeting #123																			S3-252805

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Updates on storage and processing of credentials requirements on the device

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IOT_Sec

Comments

In SA#108, the following guidance for AIoT device credential storage was endorsed:

1) 	for Rel-19, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network) e.g. SNPN, and the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501). This bullet means that no interconnection exists between AIoT systems and PLMNs ;

2) 	For Rel-20, if the AIoT system is defined as public network i.e. PLMN, the AIoT device credentials storage shall use UICC.

NOTE 1: Revisiting the above requirements in bullet 1 and/or 2 can be anticipated in future releases following the normal working procedures including in Rel-20.

NOTE 2: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP.

To accommodate the above guidance, especially the bullet 1) for Rel-19, it is proposed to update the scope and the security requirements. It is also proposed to remove the Editor’s Notes in the scope and the security requirements as they are no longer applicable based on the proposed updates.

Proposed Changes

# * * * * First change * * * *

# 1	Scope

The present document specifies the security and privacy aspects of AIoT services in the 5G System (5GS), complying to the requirements in TS 22.369 [4], applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [3], and based on the architecture defined in TS 23.369 [2]. The AIoT system is defined as private network (i.e., SNPN) in the present document.

Security features for AIoT services include:

1. Network Layer Authentication between AIoT device and 5G core

a. AIoTF is the endpoint in the 5G core

b. Credentials are securely stored in the ADM on the network side

NOTE 1: The credentials are assumed to be stored in a secure environment in the ADM. How this is realized is left to implementation. The requirements will reflect this.

c. Secure storage and processing of credentials in the AIoT device.

NOTE 2: For SNPN deployment the storage of the credentials of non-AKA based methods is out of scope as described in TS 33.501[5] Annex I 2.2.

d. Security aspects of the storage of the credentials at the ADM

2. Confidentiality, anti-replay and integrity protection of information during AIoT service communication

3. Privacy of AIoT device identifiers using the AIoT Temp ID.

4. Security to protect the permanent disabling RF transmission capabilities of AIoT device(s).

# * * * * Second change * * * *

#### 4.2.1.1	Secure storage and processing of credentials

The long-term credentials used for authentication shall be securely stored and processed on the AIoT device.

NOTE x: how to realize the above requirement for long-term credentials is outside the scope of the specification.

# * * * * End of change * * * *