3GPP TSG-SA3 Meeting #123																			S3-252811

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Updates on the command protection

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IOT_Sec

Comments

This contribution proposes to update the command protection to address the following editor’s notes:

Editor’s Note: The derivation and retrieval of the KAIOTF key are FFS.

Editor’s Note: How to prevent key stream reuse is FFS.

Editor’s Note: whether the KAIOTF key is fresh for each round of inventory-command procedure is FFS.

Editor’s Note: input key is FFS.

Editor’s Note: The details of replay protection are FFS.

The above Editor’s Notes are deleted by the following design: the AIoT device and AIOTF derive a command protection key (i.e., KCommand) based on the KAIOTF, RANDAIOT_n and RANDAIOT_d. This key is derived in step 2 and used to derive integrity algorithm and ciphering algorithm keys (i.e., KCommand-int and KCommand-enc).

In addition, it is assumed that only one command procedure (i.e., a single roundtrip of Command Request and Command Response messages) is performed after the inventory procedure. By deriving a fresh KCommand for each command procedure, security of encryption and integrity protection is ensured (addressing the Editor’s Note on the keystream reuse) and potential replay attacks are prevented.

Proposed Changes

# * * * * First change * * * *

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The inventory procedure as described in clause 5.2.2 shall be performed. The AIoT device and AIOTF derive KCommand based on the KAIOTF, RANDAIOT_n, and RANDAIOT_d as specified in Annex A.yy. The encryption key,  KCommand-enc and integrity protection key, KCommand-int are derived as specified in Annex A.yz.

3.	The AIOTF shall construct a NAS Command Request and protect the message based on the keys for integrity and ciphering algorithms.. The AIOTF shall send the Command Request containing the protected NAS Command Request to NG-RAN.

Editor’s Note: the selection of confidentiality and integrity algorithms by the network is FFS

4.	The NG-RAN shall send a R2D message containing the protected NAS Command Request as specified in TS 38.300 [3] and TS 38.391 [6] .

5.	The device shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the configured encryption and integrity algorithms.

6.	The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the Command Response containing the protected NAS Command Response to the AIoTF.

8-9.	The AIOTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

### 5.3.3	Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the KCommand-int key.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

### 5.3.4	Input parameters to ciphering algorithm

The input parameters for the ciphering algorithms shall be the same as the ones used for integrity protection as described in clause 5.3.3, with the exception that a different key, KCommand-enc is used as KEY, and there is an additional input parameter, namely the length of the key stream to be generated by the ciphering algorithms.

# * * * * Second change * * * *

# A.zx	Function FB

The AIoT key derivation function is defined as follows:

Output = FB(key, input parameters)

Editor’s Note: the details of function FB is FFS.

# * * * * Third change * * * *

# A.yy	Command protection key derivation

When deriving the command protection key (i.e., KCommand), the following parameters are used as input to the FB as defined in Annex A.zx:

-	RANDAIOT_n;

-	RANDAIOT_d

The input key shall be KAIOTF.

# * * * * Third change * * * *

# A.yz	Algorithm key derivation functions

When deriving keys for integrity and ciphering algorithms (i.e., KCommand-int and KCommand-enc) from KCommand in the AIOTF and AIoT device, the following parameter shall be used as input to the FB as defined in Annex A.zx:

- Algorithm name

The input key shall be KCommand.

# * * * * End of change * * * *