3GPP TSG-SA3 Meeting #123		                                                                   S3-252822

Goteborg, Sweden, 25 – 29 August 2025

# Source:	China Mobile, Johns Hopkins University, Deutsche Telekom

# Title:	New Study on feasibility of decentralized trust enablement for 6G

# Document for:	Approval

# Agenda Item:	6.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New study on feasibility of decentralized trust enablement for 6G

Acronym:FS_DecenTrust

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

N/A

3	Justification

In TR 22.870, the security issues of operators struggling with the bilateral administration of security to enable interconnection and roaming with other operators has been studied. Similar issues also exist in intra-PLMN cases where different security domains in the core network rely on NDS/IP and NDS/AF to provide transport layer security protection mechanisms. In both inter-PLMN and intra-PLMN interconnections, digital certificates are usually used as identity identifiers and IPSec protocol is used for securing the communication between the two domains. This requires both CA (Certificate Authorities) institutions of each operator/entity at the border of the security domains to crossly trust and certify each other. Frequent cross-domain certification brings about network latency and management cost, as well as threats of single point of failure. What’s more, it is difficult to establish a common root of trust for cross domain communication.

Thus, an efficient manner of establishing trust among inter-PLMN and intra-PLMN in 6G needs to be studied. For example, the traditional centralized trust infrastructure could be replaced with a decentralized trust foundation within and across the trust regions. With that, the 6G core network would fundamentally eliminate single point of failure and reduce the cross-domain security management burden in security certification establishment.

In addition, the increasing system complexity, driven by hybrid cloud deployments and multi-layered architectures, demands a re-evaluation of current trust and security mechanisms. It is critical to explore the possibility of decentralized trust framework and technologies that can address existing gaps while future-proofing security for emerging challenges, by offering resilience for distributed cloud and edge deployments and efficient cross-domain trust establishment.

The feasibility study of decentralized trust could serve as a security infrastructure for securing intra-PLMN NF communications, multi-domain trust establishementin intra-PLMN, inter-PLMN interconnection, and emerging decentralized applications, etc.

4	Objective

The objective of this study item is to:

- WT1:Study the feasibility of decetralized trust mechanisms to eliminate single-point-failure and cross-domain security management burden.

  - For inter-PLMN case, study the efficient authentication mechanismsbetween operators.

  - For intra-PLMN case, study the feasibility of fundamental authentication framework serving the trust establishment among network elements in the same or different security domains.

NOTE：The GSMA and/or other industry organizations may undertake similar topics in similar timeframes. However, 3GPP is responsible for achieving the objectives of this study. Potential liaison communications might be triggered during the study.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33.XXX | Study on feasibility of decentralized trust enablement for 6G | Sep 2026 TSG#117 | Dec 2026 TSG#118 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA3

8	Aspects that involve other WGs

None.

9	Supporting Individual Members

Supporting IM name

China Mobile

# Johns Hopkins University

# Deutsche Telekom

# AT&T

Boost Mobile Network

Verizon

Vodafone

China Unicom

Lenovo

Charter Communications

MITRE

US NSA

US OTD

CAICT