3GPP TSG-SA3 Meeting #123	S3-252915

Goteborg, Sweden, 25 – 29 August 2025

# Source:	IIT Bombay

# Title:	pCR to TS 33.502 - Security related Events for Authorization and Authentication

# Document for:	Approval

Agenda Item:	5.1.1

# 1	Decision/action requested

It is proposed to approve this pCR for incorporation into the draft TS 33.502

# 2	References

[1]	3GPP TS 33.502 v0.0.1

# 3	Rationale

Based on approved WID (SP-250876), the working task needs to specify the general requirements for security events handling and collection.

In WT2, it will specify the events that need to be reported including the following.

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

It is proposed to add security requirements for configuration of security related events.

# 6	Security related Events

Editor’s Note: This clause addresses the list and description of the events as well as naming convention for the events.

* * * First Change * * * *

## 6.x	Security events related to authorization and authentication

Authentication events occur during mutual TLS procedures when an entity within the trusted domain or outside the trusted domain establishes a connection with an entity within the network. Authenticating a compromised entity or user can lead to malicious attacks. Also, only authorized users or entities should be allowed to access the services or information they have privilege for. Any unprivileged access again leads to misuse of resources and other malicious attacks. A service access request is authenticated and authorized in accordance with the issued access token and NRF policy evaluation.

Security events related to authentication and authorization need to be collected to monitor abnormal behavior. Logging both successful and unsuccessful events related to authentication and authorization is essential for traceability and accountability, detection of privilege escalation attempts and compromised NFs.

Absence of such logs hinders with the process of security analysis due to lack of validation data.

Security events related to successful and failed authentication can have the following naming convention:

- NF_AUTHN_SUCCESS

- NF_AUTHN_FAILURE

NF_AUTHN_SUCCESS security event log shall include the following details:

- NF Consumer ID, NF Producer ID, nfInstanceID

- Certificate information of the Consumer (subjectAltName, subject DN, issuer)

- Timestamp of the authentication

- Source and target IP addresses

NF_AUTHN_FAILURE security event log shall include the following details

- NF Consumer ID, NF Producer ID, nfInstanceID

- TLS certificate details, including expiration time, subject DN, unsupported operator CA, serial number, and public key information

- Reason for failure (e.g., CERT-EXPIRED, SAN-MISMATCH, UNSUPPORTED-CA)

- Timestamp of the failed attempt

- Source and target IP addresses

Authentication can use the following security event log template:

NF_AUTHN_SUCCESS::<Producer-ID>::<Consumer-ID>::<CertificateDetails>::<Timestamp>

NF_AUTHN_FAILURE::<Producer-ID>::<Consumer-ID>::<CertificateDetails>::<Reason-Code>::<Timestamp>

Authorization events occur when the NF service producer verifies the access token issued by the NRF and evaluates whether the NF Consumer is permitted to invoke the requested service.

Security events related to successful and failed authorization can have the following naming convention:

- NF_AUTHZ _SUCCESS

- NF_AUTHZ _FAILURE

NF_AUTHZ_SUCCESS security event log shall include the following details:

- NF Consumer ID, NF Producer ID, nfInstanceID;

- Access token claims validated (audience, scope, expiration, nfType);

- Authorized service name and service type;

- Requested and permitted actions;

- Timestamp of the authorization decision;

- Source and target IP addresses.

NF_AUTHZ_FAILURE security event log shall include the following details:

- Token claims information: nfinstance ID of Consumer or issuer (NRF), NF Consumer ID, NF Producer ID, issuer, scope, expiration, nfType, PLMN ID, NRF issuer signature validity, expected NF service name

- Authorization decision details from the NRF (including policy denial reason);

- Requested resource and attempted actions;

- Reason for failure (e.g., TOKEN-INVALID, TOKEN-EXPIRED, POLICY-DENIED);

- Timestamp of the failed attempt;

- Source and target IP addresses

Authorization can use the following security event log template:

NF_AUTHZ_SUCCESS::<Producer_ID>::<Consumer_ID>:>::<TokenClaimInformation>::<Timestamp>

NF_AUTHZ_FAILURE::<Producer_ID>::<Consumer_ID>::<TokenClaimInformation>::<Reason_Code>::<Timestamp>

* * * End of Changes * * * *