3GPP TSG-SA3 Meeting #126	S3-260886

# Goa, India, 9 – 13 February 2026	revision of S3-260690

Source:	Ericsson

Title:	Pseudo-CR on New solution for KI#2

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.2.0

Work Item:	FS_AEAD

Comments

This document proposes a solution for the AEAD algorithms input and output parameters.

* * * First Change * * * *

## 6.Y	Solution Y: AEAD algorithm parameters

### 6.Y.1	Introduction

This solution addresses the requirement of key issue #2 (AEAD algorithm interface) by following RFC 5116 [6] defined AEAD interface (it is called generic AEAD interface throughout the document) and 3GPP related parameters that are used so far in the encryption and integrity protection of the air interface.

Following the generic AEAD interface is useful in terms of crypto agility which means the protocol does not require changes even if the used crypto algorithm is changed, which makes replacement of AEAD algorithms in the future easier.

The solution first presents the generic AEAD interface documented in RFC 5116 [6], then shows how this interface can be used in 3GPP by taking the commonly used 3GPP related parameters. Finally, the solution analyses how the 256-NCA4/5/6 algorithms can be treated as AEAD algorithms having generic AEAD interface.

### 6.Y.2	Solution details

#### 6.Y.2.1	Generic AEAD interface

According to RFC 5116 [6], an AEAD algorithm takes a fixed length key (K), a fixed length Nonce (N), an arbitrary length plaintext (P), an arbitrary length associated data (A) as input and produces a ciphertext (C) as output. The corresponding decryption algorithms takes the key (K), the nonce (N), the ciphertext (C), and the associated data (A) as input and outputs the corresponding plaintext if the integrity verification is successful, otherwise returns a failure. Figure 6.Y.2.1-1 depicts the generic parameters of AEAD encryption and AEAD decryption operations.

Figure 6.Y.1-1: RFC 5116 AEAD interface

#### 6.Y.2.2	How to use generic AEAD interface in the 3GPP system

##### 6.Y.2.1.1	Potential 3GPP specific parameters

Assume that potential 3GPP specific parameters to be used in the AEAD operations are the following.

-	KEY: a 256-bit key.

-	COUNT: 32-bit value.

-	BEARER: a 5-bit bearer identity.

-	DIRECTION: a 1-bit direction of the transmission. It is 0 for uplink and 1 for downlink.

-	EXTRA_IV: a 48-bit value.

-	PLAINTEXT: an arbitrary length parameter that needs to be encrypted and integrity protected. It is an input in the encryption mode and an output in the decryption mode. If integrity verification fails in the decryption mode, the output PLAINTEXT becomes an error.

-	ASSOCIATED DATA: an arbitrary length input that needs integrity protection.

-	CIPHERTEXT: an arbitrary length parameter. It is an input in the decryption mode and an output in the encryption mode. It contains both the encryption result of the PLAINTEXT and integrity check value of the PLAINTEXT and ASSOCIATED DATA.

Editor’s Note: The parameters will be specified during the 6G study and normative work. The list above is just an example.

##### 6.Y.2.1.2	Mapping of 3GPP specific parameters to the generic AEAD parameters

Figure 6.Y.2.1.2-1 and 6.Y.2.1.2-2 illustrate the use of the generic AEAD algorithm in encryption and decryption mode in the sender and receiver side, respectively, in 3GPP. The mapping of potential 3GPP specific parameters and generic AEAD algorithm parameters is shown below:

-	Generic AEAD parameter K: is the potential 3GPP parameter KEY.

-	Generic AEAD parameter N: is constructed from the potential 3GPP parameters COUNT, BEARER, DIRECTION, and EXTRA_IV.

-	Generic AEAD parameter P: is the potential 3GPP parameter PLAINTEXT.

-	Generic AEAD parameter C: is the potential 3GPP parameter CIPHERTEXT.

-	Generic AEAD parameter A: is the potential 3GPP parameter ASSOCIATED DATA.

Figure 6.Y.2.1.2-1: Usage of generic AEAD algorithm in encryption mode in 3GPP

Figure 6.Y.2.1.2-2: Usage of generic AEAD algorithm in decryption mode in 3GPP

##### 6.Y.2.1.3	Analysis on how the 256-NCA4/5/6 algorithms can be treated as AEAD algorithms having generic AEAD interface

Figure 6.Y.2.1.3-1 and 6.Y.2.1.3-2 illustrate how to construct AEAD algorithms having generic interface from the 256-NCA4/5/6 algorithms. The mapping of parameters is presented below:

-	256-NCA4/5/6 parameters EXTRA_IV, COUNT, BEARER, DIRECTION are extracted from generic AEAD parameter N.

-	256-NCA4/5/6 parameter KEY is the generic AEAD parameter K.

-	256-NCA4/5/6 parameter IBS is the generic AEAD parameter P in the encryption mode and the generic AEAD parameter C after removal of the integrity protection bits in the decryption mode.

-	256-NCA4/5/6 parameter OBS is the generic AEAD parameter P in the decryption mode and ciphertext part of the generic AEAD parameter C in the encryption mode.

-	256-NCA4/5/6 parameter MAC is the integrity protection part of the generic AEAD parameter C.

-	256-NCA4/5/6 parameter AAD is the generic AEAD parameter A.

-	256-NCA4/5/6 parameter MAC_BYTES is set to 4 (assuming that 6G uses 32-bit MAC).

-	256-NCA4/5/6 parameter MODE is set to 0 (resp. 1) in the encryption mode (resp. decryption mode).

-	256-NCA4/5/6 parameter S_LENGHT and ADD_LENGTH are the byte lengths of the generic AEAD parameters P and A, respectively.

In the decryption mode, the MAC is extracted from the generic AEAD parameter C and if the computed 256-NCA4/5/6 parameter MAC equals to the extracted MAC, then the P becomes the OBS, otherwise P becomes an error. Note that in the encryption mode, the generic AEAD parameter C includes both the 256-NCA4/5/6 parameter OBS (ciphertext part of the 256-NCA4/5/6 output) and the 256-NCA4/5/6 parameter MAC (integrity protection part of the 256-NCA4/5/6 output).

Figure 6.Y.2.1.3-1: Usage of 256-NCA4/5/6 algorithms as AEAD Encryption Algorithm

Figure 6.Y.2.1.3-2: Usage of 256-NCA4/5/6 algorithms as AEAD Decryption Algorithm

### 6.Y.3	Evaluation

TBD

* * * End of Changes * * * *