3GPP TSG SA WG3 (Security) Meeting #81	S3-152309

9-13 November 2015 Anaheim(US)		revision of S3-13abcd

Source:	Huawei, HiSilicon

Title:	Modify test case of 5.2.2.3.2

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add an execution step and the corresponding results in 5.2.2.3.2 NAS integrity algorithm selection and use of 33.116.

- Introduction

The expected results of 5.2.2.3.2 NAS integrity algorithm selection and use in 33.116 is going to obtain the evidence on checking message authentication code in MME after receiving SECURITY MODE COMPLETE message. However, the checking evidence is hard to catch, for MME will continue to follow the steps in 33.401 if the result is right which leaves nothing to prove the MME has checked the MAC. Thus, we approved to add a new execution Steps which will send a wrong MAC. The check could be proved to exist when the MME refuses to go on the following steps.

- Proposed pCR

***	BEGIN OF FIRST CHANGE	***

5.2.2.3.2 	NAS integrity algorithm selection and use

Requirement Name: NAS integrity algorithm selection

Requirement Reference: TBA

Requirement Description: "The MME shall protect the SECURITY MODE COMMAND message with the integrity algorithm, which has the highest priority according to the ordered lists." as specified in TS 33.401, 7.2.4.3.1."

NOTE: 	The text in TS 33.401, clause 7.2.4.3.1 is somewhat incomplete. It should properly read: "…which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities."

Threat References: TBA

Security Objective References: TBA

Test Case:

Purpose:

Verify that NAS integrity protection algorithm is selected and applied correctly.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

The MME sends the SECURITY MODE COMMAND message. The UE replies with the SECURITY MODE COMPLETE message

Expected Results:

- The MME selects the integrity algorithm which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities. The MME checks the message authentication code on the SECURITY MODE COMPLETE message.

- If the MAC in the SECURITY MODE COMPLETE is verified, the NAS integrity protection algorithm is selected and applied correctly.

- Otherwise, the Initial Context Setup Request cannot be captured if the wrong MAC is received.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot contains the operation result