3GPP TSG-SA WG3 Meeting #81 	S3-152329

9-13 November 2015, Anaheim (USA)

Source:	China Mobile

Title:	Preventing unauthorized modification of user data in cellular IOT

Document for:	Discussion and Approval

Agenda Item:	7.13

Work Item / Release:	FS_EASE_IoT/Rel-13

Abstract of the contribution:  The pCR identifies that integrity protection should be applied to user data in cellular IOT in order to prevent unauthorized modification.

Introduction

The SA3# 79 has identified that the signalling and the user data shall be encrypted using the strongest common encryption algorithm between CIoT UE and SGSN for confidentiality [1].  Whether the integrity of user data should be protected or not remains an open issue.

References

[1] S3-151504: Proposed key issue on eavesdropping for EASE_IoT

[2] Z. Shelby, K. Hartke, and C. Bormann: The Constrained Application Protocol (CoAP). RFC 7252, June 2014.

[3]	GP-140681: Traffic model for Cellular IoT (update of GP-140560)

[4]	3GPP TR 33. 821: Rationale and track of security decisions in Long Term Evolved (LTE) RAN / 3GPP System Architecture Evolution (SAE).  Release 9, June 2009.

[5]	P. Rogaway, M. Bellare, J. Black and T. Krovetz: OCB:A Block-Cipher Mode of Operation for Efficient Authenticated Encryption.

http://csrc.nist.gov/groups/ST/toolkit/BCM/documents/proposedmodes/ocb/ocb-spec.pdf

pCR

***	BEGIN CHANGES	***

- 5.x	Key Issue #x: unauthorized modification of user data

- 5.x.1	Key issue details

It is well known that the encryption does not ensure the integrity of the plaintext. This is especially true for a stream cipher. An attacker usually knows the frame structure of user data and the meaning of each data segment, such as header and payload of the COAP protocol, which is a standardized application protocol in IoT [2]. It is not so difficult for an attacker to launch a meaningful modification by altering one or more bits in the encrypted user data without breaking stream cipher.

- 5.x.2	Security threats

User data in IoT are usually modeled as low throughput and long traffic inter-arrival time [3]. The applications of IoT are usually used for the remote information retrieve or remote system control. This is quite different from the user data in real time communication whose traffic arrival is constant and continuous. Real time communication is tolerant to the packet loss or data modification because this usually does not change the content of a real time communication [4]. A user at most perceives a noise in this case. Thus we cannot deduce that the integrity of user data in CIoT shall not be protected since integrity of user data is not required in 2G, 3G and 4G networks. Thus the modification of user data in CIOT scenario is not allowed.

- 5.x.3	Security requirements

User data can be discarded at the link layer to avoid the delivery to the upper layer for further processing if data modification is detected at the link layer. This can save the energy consumption since the upper layer doesn’t need to process the compromised data packet.

Considering the efficiency, IOT applications may rely on the security schemes in the network layer to ensure data confidentiality and integrity. Correspondingly integrity protection of user data should be deployed at the link layer.

***	END OF CHANGES	***