# 3GPP TSG SA WG3 (Security) Meeting #86	S3- 170057

# 6-10 February 2017, Sophia Antipolis (France)	revision of S3-17xabc

# Source:	Huawei, Hisilicon

# Title:	Updates to Solution #8.9 Security mechanism differentiation for network slices

# Document for:	Approval

Agenda Item:	8.4.8

# 1	Decision/action requested

Update the solution in #8.9 for clarification and remove the ENs.

# 2	References

(Reference - in list form - should be made to previous related SA5/3GPP/etc. documents.)

(For changes against a draft TS/TR, a pseudo CR - a.k.a. pCR - will be provided using this Tdoc template. In this case, the number, name and version of the draft TS/TR used as base must be provided and the version must be the latest available version of the draft TS/TR.)

[1]	3GPP TR 33.899v0.6.0 (2016-11) Study on the security aspects of the next generation system

# 3	Rationale

Update the solution in #8.9 for clarification and remove the ENs.

# 4	Detailed proposal

#### It is proposed to approve the following changes for inclusion in TR 33.899

#### pCR

#### *** BEGIN CHANGES ***

#### 5.8.4.9	Solution #8.9: Security mechanism differentiation for network slices

##### 5.8.4.9.1 Introduction

##### This solution addresses the key issue #8.2: Security mechanism differentiation for network slices.

##### 5.8.4.9.2 Security mechanism differentiation in authentication

The 3GPP System should be able to support multiple authentication protocols. Different slices may use different authentication protocols. Among all the authentication protocols used in a 3GPP network, a UE may use one or a few authentication protocols. An AU, dedicated for a slice or common to multiple slices may also use one or more authentication protocols and may not support all the authentication protocols used in a 3GPP network. In cases that a UE supports multiple authentication protocols, the UE should report its authentication capabilities (e.g. through reporting the identifiers of supported authentication protocols) to the network, and the network should select AU(s), either dedicated for a slice or common to multiple slices, for UE authentication to slices, taking into consideration the UE’s authentication capabilities. Furthermore, the network should select an authentication protocol from those supported by the slice to use, if the UE supports multiple authentication protocols.

The figure below illustrates AUSF selection and authentication protocol selection in an attach procedure.

# Figure 5.8.4.9.2-1 AU selection during attach

The description of above procedure is as below:

- A UE sends an attach request to SEAF. The request contains the UE’s authentication capabilities, i.e., the authentication protocols the UE supports.

- SEAF sends an AU selection request to SPCF. The request contains the UE’s authentication capabilities.

NOTE: 	Please refer to clause 5.2.1.2 for SPCF definition SPCF refers to Security Policy Control Function, responsible to AUSF selection, security algorithm selection etc. SPCF is formally defined in this TR.

Editor’s note: It is FFS whether the SPCF should be standardized and needed to be centralized position.

Editor’s note: It is FFS where the SPCF is physically located.

- SPCF selects an AU considering the UE’s authentication capabilities. If the UE’s authentication capabilities include multiple authentication protocols, SPCF needs to select a protocol as well.

- SPCF returns the selected AUSF’s ID to SEAF.

- With selected AUSF’s ID, SEAF forwards the UE’s attach request to the selected AUSF.

- UE and AUSF authenticates to each other. If the UE supports multiple authentication protocols, the authentication protocol is the one selected by SPCF in step 3.

##### 5.8.4.9.3	Security mechanism differentiation in confidentiality and integrity protection

Different types of slices are optimized for different services. They may have different requirements on the encryption and integrity protection algorithms of UE-CN signalling, UE-AN signalling and/or user plane traffic. For example, an eMBB slice may prefer a heavy encryption algorithm with a longer key length, while an mIOT slice may prefer a lightweight encryption algorithm and a shorter key length. In addition to the encryption algorithms and key lengths, slices may differ in integrity algorithms and/or key re-keying/refreshing settings as well. Therefore, for UE-CN signalling, UE-AN signalling and user plane traffic, different slices may have different security policies on the selection of encryption algorithms, the selection of integrity algorithms, key lengths and key re-keying/refreshing settings. Further, as a slice may provide several kinds of services, or services with different settings, its security policy should also consider the security differentiation of user plane traffic inside slices as well.

The encryption algorithms and integrity algorithms should be determined by the network. Usually, a key length is closely related to an encryption algorithm. Therefore, the selection of encryption algorithms and key lengths may be performed together by SPCF.

The selection of encryption algorithms for UE-CN signalling and UE-AN signalling should be performed by the SPCFs of CN and AN respectively after a successful mutual authentication between UE and AUSF. The security algorithm selection for user plane traffic should be performed after the UE is authorized to transmit and receive data. This selection may be implemented inside a slice, and the mechanism could be customized for different slice types.

If the selection of encryption/integrity algorithms and key lengths is dedicated for one slice, it should comply with the slice’s security policy, of which SPCF must be aware. If the selection is for multiple slices (e.g. in cases selecting algorithms for a secure channel shared by multiple slices), it should comply with the security policies of all these slices and SPCF must be aware of all these slices’ security policies.

Editor's note: further clarification is needed for the selection for multiple slices.

Besides, the selection of security policies should also consider a UE’s security capabilities and the network’s security capabilities. The figure below illustrates the selection of encryption/integrity algorithms and key lengths.

# Figure 5.8.4.9.3-1 Encryption and integrity algorithm selection for signalling protection

The description of above procedure is as below:

- SPCF obtains one or more security policies from the slice manager responsible for the security policy control.

NOTE: 	Slice managing function is an NF maintaining updated security policies for encryption algorithms, integrity algorithms, and key lengths and distribution of the security policies

- UE and AUSF authenticate each other.

- AUSF notifies SPCF that UE has been authenticated.

- SPCF selects an encryption algorithm, integrity algorithm and key lengths considering the slice security policy, UE’s security capabilities and network’s security capabilities.

- SPCF notifies UE the selection results (Security Mode Command in LTE could be reused here).

- UE acknowledges the selection results.

##### 5.8.4.9.4	Evaluation

#### *** END OF CHANGES ***